# ISRO WORK 2

Perfect 👍 Madhu — the PDF you uploaded describes a NAND Flash–based Onboard Solid State Recorder (SSR) developed by U R Rao Satellite Centre (URSC), ISRO.
Let’s go through it step by step, clearly explaining each part for your understanding (from an embedded systems point of view):

🌌 1. What is the Nand Flash–Based Onboard Solid State Recorder?

An Onboard Solid State Recorder (SSR) is a data storage device used in satellites to store payload and telemetry data.
Think of it like a “space-grade SSD” — it records data from sensors or payloads and later transmits (playbacks) that data to Earth when communication links are available.

⚙️ 2. What is NAND Flash Memory?

- NAND Flash is a non-volatile memory — it keeps data even when power is off.

- It’s the same basic technology used in USB drives, SD cards, and SSDs.

- It is lighter, faster, and denser (more data per unit area) than traditional magnetic storage or NOR Flash.

However, for space applications, NAND Flash needs extra protection because:

- It can suffer from bit errors (due to radiation, temperature, etc.).

- Its reliability degrades over time with write/erase cycles.

ISRO overcame these reliability issues through error correction, redundancy, and fault-tolerant design.

🚀 3. Why is it useful for spacecraft?

Satellites generate huge volumes of data from cameras, sensors, and instruments.
But they can’t always transmit data instantly (because the ground station isn’t always in view).
So the SSR stores all that data onboard until it can be downloaded to Earth.

Because NAND Flash is:

- Low power → great for power-limited satellites.

- High density → can store terabits of data.

- Lightweight and compact → saves mass (critical in space design).

🧩 4. Major Specifications (Explained)

| Feature | Explanation |
| --- | --- |
| Space-qualified flash memory SSR | The flash chips and design are tested and qualified for harsh space environments (radiation, vacuum, temperature). |
| Single-board or multi-board configurations | The design can use one or several boards depending on required capacity or redundancy. |
| Scalable from 176 Gb to 700 Gb per board | Each board stores between 176 and 700 gigabits (≈ 22 GB to 87.5 GB). |
| Up to 2.8 Tb for multi-board configuration | Multiple boards can be combined for a total capacity of about 350 GB. |
| Record / Playback / Near real-time operations | It can record data, replay stored data, or stream data directly (near real-time). |
| File-based data storage / retrieval | Data is organized as files — easier to manage, access, and transmit specific data. |
| Command & Control through 1553 interface | Uses the MIL-STD-1553 standard — a reliable, dual-redundant communication bus widely used in spacecraft and aircraft for control and telemetry. |

🧠 5. Key Advantages

- Non-volatile memory: Keeps data safe even after power loss.

- High density: Can store large payload data like satellite images or video.

- Low power: Ideal for long-duration missions.

- Rugged and radiation-tolerant design: Ensures reliability in space.

- Scalable and modular: Same design can be reused for small or large missions.

🧰 6. Technology Transfer

ISRO (URSC) offers this technology for transfer to Indian industries that:

- Have experience in electronics or embedded hardware.

- Can handle manufacturing, integration, and testing of such systems.

- Are interested in space-grade data handling or storage solutions.

To acquire it, industries must provide details of their facilities, current work, and capabilities to URSC for evaluation.

🛰️ 7. Real-world Applications

- Earth observation satellites: Store high-resolution images or video data.

- Scientific payloads: Save experimental data during orbit.

- Communication satellites: Buffer large amounts of signal data.

- Deep space missions: Where continuous communication isn’t possible.

Would you like me to draw a block diagram of how this Onboard Solid State Recorder (SSR) works (showing NAND Flash, controller, 1553 interface, etc.)?
That visual will make the architecture and data flow clearer from an embedded systems viewpoint.

Got it 👍 Madhu — let’s break “Technology Transfer” down step by step so you clearly understand what it means in ISRO’s context.

🚀 What is “Technology Transfer”?

“Technology Transfer” means ISRO is sharing or licensing its in-house developed technology with private Indian companies so they can manufacture or use it themselves.

In this document, ISRO’s U R Rao Satellite Centre (URSC) developed a NAND Flash–based Onboard Solid State Recorder (SSR).
Now, instead of making every unit themselves, ISRO wants to allow capable Indian industries to produce or further develop this technology.

🏭 Why does ISRO do this?

ISRO develops many advanced technologies (like satellite subsystems, sensors, onboard computers, etc.).
To boost India’s space industry, ISRO:

- Transfers those technologies to private industries,

- So that those industries can mass-produce, improve, or use them in their own products.

This helps:

- ISRO → focus on research and new missions.

- Industry → get ready-made, space-tested designs to build from.

- India → become more self-reliant in space hardware.

🧩 What kind of industries can apply?

Only Indian companies with the technical capability to handle such high-end work can get it.
Specifically, companies that have:

- ⚙️ Experience in electronics or embedded hardware design —
Because the SSR involves digital electronics, memory management, and controller design.

- 🔧 Facilities for manufacturing, integration, and testing —
They must have labs, clean rooms, and test setups to assemble and verify space hardware.

- 🛰️ Interest in space-grade data systems —
The company should plan to use or supply this technology for satellites or related missions.

📄 What do they need to do?

Interested industries must write to URSC-ISRO, providing:

- Details about their current business or projects.

- Description of their infrastructure (like labs, test equipment).

- Description of their technical team and expertise.

URSC then evaluates whether the company is capable of handling the technology responsibly.

🔑 Summary (in your words)

| Concept | Meaning |
| --- | --- |
| Technology Transfer | ISRO shares its satellite technology design and know-how with private Indian companies. |
| Purpose | To let capable companies manufacture or enhance ISRO technologies. |
| Who can apply | Indian industries with embedded/electronics expertise and testing facilities. |
| How to apply | Write to URSC-ISRO with details about activities, infrastructure, and capability. |
| Example | A company could license this NAND Flash–based SSR design and produce it for future satellites. |

Would you like me to show you how such a Technology Transfer process works step-by-step (from applying → evaluation → signing agreement → manufacturing)?
That will make it even clearer with a small example scenario.

Excellent question, Madhu 👏 — this is the right way to understand satellite systems deeply!

Let’s go step-by-step so you get a clear idea of what payload and telemetry data mean — and how they differ — using simple examples.

🛰️ 1. First — Recap: What is an Onboard Solid State Recorder (SSR)?

An Onboard Solid State Recorder (SSR) is like a space-grade “hard drive” or memory system used inside a satellite.
It stores all the data the satellite collects — both payload data and telemetry data — until that data can be sent to Earth.

It uses NAND Flash memory, which means it’s:

- Non-volatile (keeps data even when power is off),

- Compact and lightweight,

- Suitable for power-limited, radiation-exposed environments like space.

📦 2. What is Payload Data?

Let’s break it down simply.

🧠 Meaning:

Payload data = The main mission data that the satellite was launched to collect.
It’s the useful information that scientists or engineers on Earth actually want.

🛰️ Example situations:

| Type of Satellite | Payload | Payload Data Example |
| --- | --- | --- |
| Earth Observation | Camera / imaging sensor | Photos or videos of Earth’s surface |
| Communication Satellite | Transponders | User communication signals (TV, phone, internet) |
| Weather Satellite | Radiometers, IR sensors | Cloud temperature, humidity data |
| Scientific Satellite | Particle detector, telescope | Space radiation readings, star images |

So, payload data is mission-specific — it’s the main reason the satellite exists.

💡 In short:

Payload data = mission output.

⚙️ 3. What is Telemetry Data?

🧠 Meaning:

Telemetry data = Health and status information of the satellite itself.
It’s used to monitor how well the satellite is functioning — not about Earth or space, but about the satellite’s own systems.

⚙️ What it includes:

- Power system voltage and current

- Temperature of components

- Status of solar panels, batteries, gyros

- Memory usage, CPU load

- Orientation and position information

- Any warnings or fault signals

This data is usually continuously sent to Earth (in small amounts) so that ISRO’s control center can keep the satellite safe.

💡 In short:

Telemetry data = satellite’s health report.

🔍 4. The Key Difference

| Aspect | Payload Data | Telemetry Data |
| --- | --- | --- |
| Purpose | Main mission data | Satellite’s health and performance |
| Generated by | Payload instruments (camera, sensors, etc.) | Onboard systems (power, thermal, communication) |
| Example | Image of a cyclone | Battery voltage = 3.7V, temp = 45°C |
| Stored in SSR | Yes, usually large size (GBs or TBs) | Sometimes stored in small logs for safety |
| Transmitted to Earth | When ground station is visible or on schedule | Continuously or periodically, small packets |
| Importance | Used for mission results or analysis | Used for control and maintenance of the satellite |

🎯 5. How SSR handles both types of data

In a satellite:

- Payload data comes in huge chunks → recorded and compressed → stored in the SSR until a downlink window.

- Telemetry data is smaller → often sent immediately → sometimes also logged in SSR for backup or post-event analysis.

So the SSR acts like a black box:

It stores everything — both the “main work” data and the “self-health” data — safely in non-volatile memory.

🧩 Analogy (to make it simpler):

Imagine a GoPro camera drone:

- The payload data = video footage from the camera.

- The telemetry data = battery level, GPS location, altitude, motor temperature.

Both are important — one is for your mission, the other ensures your drone doesn’t crash.
Similarly, in satellites, both data types are recorded.

Would you like me to show you a diagram of how payload and telemetry data flow to the SSR inside a satellite (with blocks like payload sensors → data bus → SSR → ground station)?
That visual will make the distinction crystal clear.

Excellent question, Madhu 👏 — you’re now thinking like an embedded systems engineer working in satellite data handling.

Let’s go step by step so you understand the architecture, components, connections, and communication paths of an Onboard Solid State Recorder (SSR) used in spacecraft.

🛰️ 1. The Role of an SSR in a Satellite

An Onboard Solid State Recorder (SSR) is the data hub and storage unit of a satellite.
It receives data from:

- Payloads (sensors, cameras, instruments),

- Onboard computers (for telemetry),
stores it safely in non-volatile NAND Flash memory,
and then sends it to the ground station when communication is available.

So it sits between:
➡️ Data sources (payloads, subsystems)
➡️ Communication system (transmitter to Earth)

⚙️ 2. Main Components Inside an SSR

Here’s a breakdown of the typical hardware and functional blocks found in an SSR:

| Block | Function / Why it’s needed |
| --- | --- |
| 🧠 Controller / Processor | The “brain” of the SSR — manages recording, playback, error correction, and file organization. Usually a radiation-hardened microcontroller or FPGA. |
| 💾 NAND Flash Memory Banks | Actual storage devices. Data is written here in pages/blocks. Multiple NAND chips are used for large capacity and redundancy. |
| ⚙️ Memory Controller Interface | Controls read/write timing, wear leveling, and bad block management for NAND. Ensures data integrity. |
| 🧩 Error Detection and Correction (EDAC / ECC) | Protects against bit errors due to radiation or aging of flash. Often uses BCH or Reed–Solomon codes. |
| 🔌 Input Data Interface | Receives data from payloads (e.g., SpaceWire, LVDS, MIL-STD-1553, or custom high-speed serial interface). |
| 🖥️ Command and Control Interface | Used by the satellite’s main Onboard Computer (OBC) to configure and monitor SSR (through 1553 Bus or CAN). |
| ⚡ Power Conditioning Unit (DC–DC converters) | Converts spacecraft bus voltage (say 28V) to the SSR’s required internal voltages (3.3V, 1.8V, etc.). Ensures stable, filtered power. |
| 📡 Data Output Interface (Playback) | Sends recorded data to the Downlink System / Telemetry Transmitter when ground contact is available. |
| 🧊 Thermal and Mechanical Protection | Enclosures, heat sinks, and radiation shielding to survive space environment. |
| 🔐 File System or Data Manager | Organizes data as files (like FAT or custom file system). Helps locate and retrieve data easily. |

🧠 3. Internal Architecture (Simplified)

Here’s a text-based diagram:

+----------------------------+

|   Onboard Solid State      |

|         Recorder (SSR)     |

+----------------------------+

|                            |

|   +--------------------+   |

Payload →→ |  Input Interface     |   |

Telemetry→ | (LVDS/SpaceWire etc)|   |

|   +---------+----------+   |

|             |              |

|     +-------v--------+     |

|     |  Controller /  |     |

|     |  FPGA / CPU    |     |

|     +-------+--------+     |

|             |              |

|     +-------v--------+     |

|     | NAND Flash      |    |

|     | Memory Banks    |    |

|     +-------+--------+     |

|             |              |

|     +-------v--------+     |

|     | ECC / EDAC     |     |

|     +-------+--------+     |

|             |              |

|   +---------v----------+   |

|   |  Output Interface  |--→|→ Downlink transmitter

|   +--------------------+   |

+----------------------------+

🔗 4. Components Connected to the SSR (Externally)

| External System | Connection Type | Purpose / What data flows |
| --- | --- | --- |
| Payload Instruments | High-speed data interface (e.g., SpaceWire, LVDS, custom serial) | Sends mission (payload) data to be recorded. |
| Onboard Computer (OBC) | Command bus (e.g., MIL-STD-1553, CAN, I2C, SPI) | Sends control commands: start recording, playback, erase, health check, etc. |
| Telemetry System | Data line | SSR may provide its own status data to be sent to Earth. |
| Communication / Downlink Transmitter | High-speed playback line | Sends recorded data to Earth during contact periods. |
| Power Distribution Unit (PDU) | Power lines | Provides regulated satellite bus power (often 28V). |

🧭 5. Where the SSR Communicates (Inside the Satellite)

Inside a satellite, the SSR acts as a data middleman between multiple subsystems:

| Source | Destination | Communication Purpose |
| --- | --- | --- |
| Payload → SSR | To store raw mission data. |  |
| OBC → SSR | To send control commands (record/start/stop/status). |  |
| SSR → OBC | To send health info or data summary. |  |
| SSR → Downlink (RF Transmitter) | To send recorded data to Earth. |  |
| SSR → Telemetry System | To log SSR status data into overall satellite telemetry. |  |
| Power Bus → SSR | To power up its circuits. |  |

So, it’s part of the Data Handling Subsystem (DHS) of the satellite.

🛰️ 6. When and How Communication Happens

| Operation | Data Flow | Description |
| --- | --- | --- |
| Recording Mode | Payload → SSR | Receives large volumes of mission data in real-time. |
| Playback Mode | SSR → Downlink | Sends stored data to ground station during visibility window. |
| Command Mode | OBC ↔ SSR | OBC sends commands (e.g., “start recording”, “erase file”), and SSR responds with status. |
| Health Monitoring | SSR → OBC → Ground | Periodically sends SSR temperature, voltage, memory status, errors. |

🧠 7. Summary (in short)

| SSR Component | Connected To | Purpose |
| --- | --- | --- |
| Controller / FPGA | NAND Flash, Interfaces | Core logic control |
| NAND Flash Memory | Controller | Data storage |
| Input Interface | Payloads | Data recording |
| Output Interface | Downlink transmitter | Data playback |
| Command Interface | Onboard Computer | Control and health monitoring |
| Power Unit | Satellite bus | Power conversion and regulation |

🔋 Analogy to Understand:

Imagine the SSR as a “smart hard drive” in a computer:

- The payload is like your camera taking pictures (big data).

- The OBC is like the operating system giving commands.

- The transmitter is like your Wi-Fi module sending files to the cloud.

- The SSR sits in between, managing, storing, and protecting all that data.

Would you like me to draw a labeled block diagram (satellite subsystems + SSR connections) so you can visualize all these components and their data paths clearly?
That’ll make it super easy to remember during interviews or project explanations.

Perfect Madhu 👏 — you’re now getting into real spacecraft system testing and onboard data handling workflows.
Let’s go step by step and build a complete, clear picture of how your NAND Flash–based SSR works when tested using:

- a 1553 Bus Simulator (for command/control)

- a Data Simulator (for payload data input)

and how the SSR operates in the three modes you mentioned:
➡️ Configuration Mode, Operation Mode, and Maintenance Mode.

⚙️ 1. Test Setup Overview

You have two main external interfaces to the SSR:

| External Interface | Role |
| --- | --- |
| 1553 Bus Simulator | Acts as the satellite’s Onboard Computer (OBC) — it sends commands and requests to the SSR. |
| Data Simulator | Acts as the Payload — it sends high-speed data streams that the SSR must record. |

So in your test:

- The 1553 interface handles control (like “start recording”, “playback file”, “erase file”).

- The data simulator handles data input/output (actual payload data).

🧩 2. SSR Major Modes of Operation

Let’s define what each mode does and what operations happen inside it.

🔧 1️⃣ Configuration Mode

Purpose:
To set up and initialize the SSR before recording or playback begins.

Actions / Operations in this mode:

| Operation | Description |
| --- | --- |
| ✅ Initialize hardware | SSR powers up, controller configures interfaces, checks NAND Flash health. |
| ⚙️ Load configuration parameters | Receive setup commands from 1553 bus (e.g., file size limits, partition table, record rate, etc.). |
| 🔢 Set operating parameters | Configure input data rate, interface mode (SpaceWire, LVDS, etc.), ECC options. |
| 🧠 Perform self-test / BIT (Built-In Test) | Check memory banks, ECC units, and communication lines for errors. |
| 📁 Create / format file system | Prepare flash partitions and file directories before recording. |
| 🔌 Handshake with Data Simulator | Ready signal exchange to ensure SSR is ready to accept data. |

End of configuration: SSR sends a “Ready for operation” status to the 1553 bus.

▶️ 2️⃣ Operation Mode

Purpose:
To perform the actual recording and playback operations (main working mode).

This mode has two sub-modes:

a. Record Mode

| Step | Function |
| --- | --- |
| 1️⃣ | Receive start command from 1553 bus (“Start Recording”). |
| 2️⃣ | Data simulator starts sending payload data via data input interface. |
| 3️⃣ | Controller writes data to NAND Flash with ECC correction and time tagging. |
| 4️⃣ | File manager organizes recorded data as files with headers (metadata, timestamps, etc.). |
| 5️⃣ | Status continuously updated to 1553 bus (record count, memory usage, temperature, etc.). |

💡 Example:

Data Simulator → SSR Input Interface → Controller → NAND Flash
Command flow: 1553 Bus → SSR Controller (to start/stop recording)

b. Playback Mode

| Step | Function |
| --- | --- |
| 1️⃣ | Receive playback command from 1553 bus. |
| 2️⃣ | SSR fetches the requested file/data block from NAND Flash. |
| 3️⃣ | Sends the data stream to Downlink system (or in test, to a Data Receiver / Analyzer). |
| 4️⃣ | Supports full-file or partial playback (based on file index or time tag). |
| 5️⃣ | Provides status via 1553 (like “Playback completed” or “Error: file not found”). |

💡 Example:

NAND Flash → Controller → Output Interface → Data Receiver
1553 bus → issues playback command + monitors status

c. Other Operations in Operation Mode

| Function | Description |
| --- | --- |
| 🧾 File management | Create, list, delete, rename, or protect recorded files. |
| 🔄 Status Telemetry | SSR reports health: memory used %, temperature, voltage, ECC corrections, etc. |
| ⚡ Data Compression (optional) | Some SSRs compress payload data before storage. |
| ⏱️ Time Tagging | Timestamp data with mission time (from OBC or internal RTC). |
| 📤 Data Verification | After recording, a quick checksum or CRC may verify integrity. |

🧰 3️⃣ Maintenance Mode

Purpose:
To perform diagnostics, health checks, and data integrity tests — usually during ground testing or after flight.

Actions / Operations in this mode:

| Operation | Description |
| --- | --- |
| 🧪 Memory Test / ECC check | Verify NAND Flash blocks, detect bad blocks, re-map if needed. |
| 🔍 Error Log Retrieval | Retrieve stored event logs (e.g., ECC errors, write failures). |
| 🧼 Data Erase / Reformat | Securely erase NAND memory or reformat partitions. |
| 🔧 Firmware Update | Load new software or FPGA bitstream through 1553 interface. |
| 📊 Performance Monitoring | Measure throughput, temperature, voltage, power consumption. |
| 🧠 BIT Execution | Built-In-Test routines to confirm system readiness before next operation. |

🛰️ 3. Summary of All Functions by Mode

| Mode | Purpose | Key Operations |
| --- | --- | --- |
| Configuration Mode | Setup and initialization | Self-test, load configs, handshake, format storage |
| Operation Mode | Main working mode | Record, playback, file management, telemetry reporting |
| Maintenance Mode | Diagnostic and service | Memory check, error log, erase, firmware upgrade |

🔗 4. Communication Overview (During Testing)

| Interface | Role | Example Communication |
| --- | --- | --- |
| MIL-STD-1553 Bus | Command & control | Mode change, start/stop record, request status, playback file |
| Data Interface (from simulator) | Payload data input | High-speed data stream during record |
| Playback Output Interface | Data output | Sends recorded file stream for verification |
| Telemetry Lines (optional) | SSR → test console | Sends periodic health data |
| Power Interface | Power supply | Regulated bus voltage to SSR board |

⚙️ 5. Other Important SSR Operations You Should Mention

| Operation | Description |
| --- | --- |
| File Directory Management | Maintain file index table — so recorded data can be quickly accessed or erased. |
| Health Monitoring | Periodic housekeeping telemetry: voltage, current, temperature, ECC errors. |
| Event Logging | Record system events (start/stop, errors, command failures) for debugging. |
| Data Verification | Optional CRC/Checksum verification after recording. |
| Watchdog Monitoring | Auto-recovery from software or hardware faults. |
| Safe Mode Handling | Switch to safe mode if over-temperature, over-current, or ECC overflow occurs. |

🧠 Summary in Simple Terms

In short, your SSR system behaves like a smart flight recorder:

- Configuration mode — Setup and prepare the system.

- Operation mode — Perform recording/playback and manage files.

- Maintenance mode — Diagnose, test, clean, or update the system.

The 1553 simulator acts like the commander, and the data simulator acts like the data source.

If you’d like, I can draw a block diagram showing:

- all connections (SSR ↔ 1553 simulator ↔ data simulator ↔ host PC),

- the 3 modes with data/control flow,
so you can use it for your lab report or presentation.
Would you like me to make that diagram next?

Excellent, Madhu — now you’re asking a system-level embedded integration question 👏.
Let’s connect everything together: SSR + MIL-STD-1553 + Data Simulator + EZ-USB FX3 — and see why FX3 is used, where it fits, and how it works in your setup.

🧠 1. What is the EZ-USB FX3?

The EZ-USB FX3 (by Cypress / Infineon) is a USB 3.0 peripheral controller — basically a high-speed bridge between USB and any custom interface.

It contains:

- A 32-bit ARM9 processor (for control firmware),

- A GPIF II (General Programmable Interface) — a flexible parallel interface that can connect to any external logic, FPGA, or microcontroller,

- A USB 3.0 link — for fast data transfer to a PC (up to 5 Gbps).

So, FX3 is perfect for data capture, testing, or high-speed streaming between a satellite board and a ground PC.

🛰️ 2. Why use EZ-USB FX3 with the SSR?

In your lab setup, the SSR is a space-grade unit designed to communicate using onboard interfaces (like 1553, LVDS, or SpaceWire).
But your test environment (PC, data simulator software, etc.) often speaks USB.

The FX3 acts as a bridge between:

- The SSR board (via FPGA or controller) and

- The PC-based test system (host software).

🧩 So, EZ-USB FX3 is used to:

| Function | Explanation |
| --- | --- |
| 🔗 Interface bridge | Connect the SSR’s data or control interface to a PC through USB 3.0. |
| ⚙️ High-speed data streaming | Capture payload data at high speed (hundreds of MB/s) for testing. |
| 💾 Playback verification | Stream recorded data from SSR memory to the PC for analysis. |
| 📜 Command forwarding | (Optionally) relay commands between the test GUI and SSR via USB. |
| 🧠 Firmware control | Run small embedded firmware to format data packets, add headers, or manage flow control. |

⚙️ 3. Where FX3 Fits in the SSR Test Setup

Let’s map your entire setup:

+----------------------+

|    Test PC (Host)   |

|  GUI + Drivers + SW |

+---------+------------+

|

| USB 3.0 (5 Gbps)

|

+---------v------------+

|   EZ-USB FX3 Board   |  ← Bridge/Interface

+---------+------------+

| GPIF II Interface

|

+-------------------v-------------------+

|          SSR Controller / FPGA        |

|  - Receives data from Payload Sim.    |

|  - Manages NAND Flash (Record/Play)   |

|  - Handles 1553 Commands              |

+-------------------+-------------------+

|

| 1553 Bus Interface

|

+---------v------------+

|   MIL-STD-1553 Sim   | ← Sends commands

+----------------------+

|

| Data input interface

|

+---------v------------+

|     Data Simulator   | ← Sends payload data

+----------------------+

🧠 4. How FX3 Works with SSR in Each Mode

🔧 Configuration Mode

- PC sends setup/configuration data via USB → FX3 → SSR controller.

- FX3 can also upload initial test scripts or configuration parameters.

- Useful for automated test setups (e.g., setting up recording parameters).

▶️ Operation Mode

- Record:
FX3 may be used to stream data into the SSR (if it’s acting as data source) or to monitor data flow for validation.

- Playback:
SSR reads data from NAND Flash → sends via controller → GPIF II → FX3 → USB → PC.
The PC receives full playback data for analysis or comparison with expected data.

🧰 Maintenance Mode

- FX3 provides diagnostic connectivity for memory test logs, ECC error logs, or firmware update via USB.

- Can also perform firmware upgrades (load new bitstream or software image into SSR controller or FX3 itself).

⚙️ 5. Communication Flow Example (Playback Test)

| Step | Action |
| --- | --- |
| 1️⃣ | 1553 simulator sends a command “Playback File #3” to SSR. |
| 2️⃣ | SSR fetches data from NAND Flash memory. |
| 3️⃣ | Data is streamed through FPGA / controller parallel bus. |
| 4️⃣ | GPIF II of FX3 captures the data stream. |
| 5️⃣ | FX3 firmware sends data packets over USB 3.0 to the host PC. |
| 6️⃣ | PC receives and stores/analyzes playback data. |

Result: You can validate throughput, data integrity, and file correctness.

⚙️ 6. Firmware Roles (FX3 Side)

The FX3 is programmed using C firmware (in eCos) or Cypress SDK.
Common firmware functions in this setup:

| Function | Description |
| --- | --- |
| 🧠 GPIF II state machine | Defines data handshaking between SSR logic and FX3. |
| 📦 USB endpoint management | Handle bulk endpoints for record/playback streams. |
| ⏱️ Flow control | Use flags to avoid overflow/underflow between SSR and USB host. |
| 🔄 Command channel | A control endpoint used for small commands or status queries. |
| 🧩 Buffer management | Double/triple buffering for sustained high-speed transfer. |

🔌 7. Why FX3 is Ideal for SSR Testing

| Feature | Benefit for SSR Tests |
| --- | --- |
| USB 3.0 SuperSpeed (5 Gbps) | Easily handles large SSR data rates (hundreds of MB/s). |
| Flexible GPIF II interface | Connects directly to FPGA/Controller without custom ASIC. |
| On-chip ARM9 | Allows custom control logic for packetization, status, etc. |
| Easy PC Integration | Works with Cypress drivers + custom Python/C GUI on PC. |
| Bus-powered option | Simplifies test setup (optional). |

🔍 8. Summary — Role of EZ-USB FX3 in Your SSR Setup

| Aspect | Explanation |
| --- | --- |
| Main Purpose | Acts as a high-speed bridge between SSR hardware and test PC. |
| Interfaces | GPIF II ↔ SSR Controller, USB 3.0 ↔ Host PC |
| Used For | Data transfer, monitoring, debugging, file verification, firmware updates |
| Modes Supported | All three (Configuration, Operation, Maintenance) |
| Direction of Data | Bi-directional (Record: PC→SSR, Playback: SSR→PC) |

💡 Simple Analogy

Think of the EZ-USB FX3 as a USB data pipeline:

It connects your SSR hardware to your computer,
letting you observe, control, or test data recording and playback operations just like how a logic analyzer connects to a PC — but much faster.

Would you like me to show a block diagram combining
SSR + 1553 simulator + Data simulator + FX3 + PC — with labeled data and command flow arrows for all three modes (record/playback/config)?
It’ll make your report or presentation on this topic crystal clear.

Excellent question, Madhu 👏 — this is exactly what you should understand deeply if you’re working hands-on with the EZ-USB FX3 in your SSR testing setup.

Let’s go step by step — we’ll cover what programming FX3 in C really means, what you can do with it, what drivers are involved, and how it connects to your SSR system.

🧠 1. First — What is “Programming the FX3 in C”?

When we say program the EZ-USB FX3 in C, we mean writing firmware that runs on its ARM9 microcontroller.

This firmware controls:

- The USB interface (to communicate with a PC),

- The GPIF II interface (to connect to your SSR controller / FPGA),

- Any other onboard peripherals (GPIOs, UART, I²C, SPI, etc.).

You write this code in C, compile it with the Cypress FX3 SDK toolchain, and load the .img binary into the FX3 device (via USB boot or SPI flash).

🧩 2. FX3 Architecture (So You Know What You’re Programming)

| Block | What it does |
| --- | --- |
| ARM9 CPU (200 MHz) | Runs your C firmware (main program). |
| USB 3.0/2.0 Controller | Handles high-speed communication with the host PC. |
| GPIF II Interface | Parallel programmable interface (connects to FPGA/SSR logic). |
| DMA Channels | High-speed data movement between USB and GPIF II without CPU load. |
| Peripherals | I²C, SPI, UART, GPIO, Timers, etc. |
| Boot Options | From USB, I²C EEPROM, SPI Flash, or internal ROM. |

⚙️ 3. What You Can Do by Programming FX3 in C

Here’s the complete list of major tasks and capabilities you can implement in firmware — especially useful for your SSR environment.

🔹 A. Configure and Control Interfaces

| Task | What you can do in C |
| --- | --- |
| USB Configuration | Create endpoints (Bulk, Interrupt, Isochronous), set packet sizes, implement custom USB device descriptors. |
| GPIF II Configuration | Design and load state machine (using GPIF II Designer tool) and control it from C code — decide how data moves to/from SSR logic. |
| GPIO Control | Configure GPIOs for handshaking (ready/busy signals between FX3 and SSR). |
| SPI/I²C/UART Setup | Communicate with external chips — e.g., to talk to SSR controller, EEPROM, sensors, or debug console. |

🔹 B. Manage High-Speed Data Transfers

| Function | Description |
| --- | --- |
| Bulk Data Streaming (USB 3.0) | Send or receive large data blocks between PC and SSR board — e.g., record/playback data streams. |
| DMA Channel Control | Link GPIF II → USB endpoints using DMA for continuous, high-throughput transfer. |
| Buffer Management | Implement double/triple buffering in C to avoid underflow/overflow. |
| Data Packet Formatting | Add headers, timestamps, CRC checks before sending data to PC. |
| Flow Control Logic | Stop/start streaming based on SSR “ready” or “busy” signals. |

✅ Example for your SSR test:
You can capture data coming from SSR’s playback interface and stream it to your PC at 300 MB/s using DMA with almost no CPU overhead.

🔹 C. Implement Custom Commands and Control Protocols

You can define your own vendor-specific USB commands (called Vendor Requests).

| What it lets you do | Example in SSR testing |
| --- | --- |
| Create a Command Channel over USB | PC sends command “Start Recording”, “Read File #3”, “Erase File” to FX3 |
| FX3 firmware decodes the command in C | Sends equivalent control signal or SPI/I²C message to SSR controller |
| FX3 sends response back to PC | Returns status like “Recording Started”, “Playback Complete” |

➡️ This is how you can control your SSR without the 1553 simulator, directly from a PC test GUI.

🔹 D. Perform Maintenance and Debug Functions

| Task | Description |
| --- | --- |
| Read/Write EEPROM or Flash | Store calibration data or configuration. |
| Firmware Upgrade over USB | Implement DFU (Device Firmware Upgrade) capability. |
| Log System Data | Send debug messages to UART or USB for test monitoring. |
| Diagnostics | Periodically check link status, throughput, or CRC errors. |

🔹 E. Interface with the Host (PC)

When you connect FX3 to a PC, it appears as a USB device.
You can make it behave as:

| USB Device Class | Purpose |
| --- | --- |
| Vendor-specific | Custom interface (typical for SSR test). |
| USB Video Class (UVC) | For streaming camera data. |
| USB Mass Storage Class (MSC) | So SSR data appears as a drive on PC. |
| CDC/Serial (UART over USB) | For command console. |

➡️ For your SSR system, you’ll likely use a vendor-specific class — so you can define your own protocol and endpoints.

💻 4. Driver Side (On the PC)

When you program FX3 firmware, you also deal with host-side drivers and software.

| Layer | Description |
| --- | --- |
| Cypress USB Driver (CyUSB3.sys) | Provided by Cypress SDK; makes your FX3 appear as a USB device in Windows. |
| CyAPI / libusb / Python Script | You can write C, C++, or Python programs on PC to send/receive data to/from FX3 endpoints. |
| Test GUI or CLI | Used to issue commands, collect playback data, or visualize logs. |

So your C firmware on FX3 talks to the driver + user application on PC through USB endpoints.

⚙️ 5. Typical FX3 Firmware Structure (C Code)

Here’s a simplified skeleton:

#include "cyu3system.h"

#include "cyu3usb.h"

#include "cyu3gpif.h"

#include "cyu3dma.h"

#include "cyu3os.h"

#include "cyu3error.h"

void MyGpifConfig(void) {

CyU3PGpifLoad(&MyGpifConfigData); // load GPIF II state machine

CyU3PGpifSMStart(RESET, ALPHA_START);

}

void MyUsbInit(void) {

CyU3PUsbStart();

CyU3PUsbRegisterSetupCallback(MySetupCallback, CyTrue);

CyU3PUsbRegisterEp0DataHandler(MyEp0Handler);

CyU3PUsbSetEpConfig(0x81, &epCfg); // IN endpoint

CyU3PUsbSetEpConfig(0x02, &epCfg); // OUT endpoint

}

void ApplicationThread(uint32_t input) {

while (1) {

// Check DMA buffers and move data between GPIF and USB

CyU3PDmaChannelGetBuffer(&dmaChannel, &buffer, CYU3P_NO_WAIT);

ProcessData(buffer.buffer, buffer.count);

CyU3PDmaChannelCommitBuffer(&dmaChannel, buffer.count, 0);

}

}

int main(void) {

CyU3PDeviceInit(NULL);

CyU3PKernelEntry();  // Start RTOS scheduler

}

➡️ This firmware sets up USB, configures GPIF, and runs a loop transferring data using DMA.

🔧 6. What You Can Practically Build for Your SSR Project

| Category | Example Feature You Can Implement |
| --- | --- |
| Data Handling | High-speed data capture from SSR to PC via USB 3.0. |
| Command Interface | Custom vendor commands for SSR configuration & mode control. |
| Firmware Update | PC → FX3 → SSR for maintenance mode reprogramming. |
| Health Monitoring | Send SSR temperature, voltage, ECC logs to PC periodically. |
| Diagnostics GUI | PC application displays throughput, link status, and SSR memory usage. |

🧠 7. In Short — What You Can Do with FX3 in C

| Domain | What You Can Do |
| --- | --- |
| Interface Control | Configure USB/GPIF/UART/SPI/I²C |
| High-Speed Data Flow | Implement DMA-based data streaming |
| Custom Commands | Define your own USB control protocol |
| Device-to-Host Communication | Transfer recorded data to PC |
| Maintenance / Logging | Firmware update, status monitoring |
| Automation | Integrate with PC tools for SSR test automation |

💡 Analogy

Think of FX3 as your data and command bridge brain:

You write C firmware to decide
what data to capture, how fast, in what format,
and how to communicate with your PC test tools.

Everything about how SSR’s data moves to/from the PC is under your control.

If you’d like, I can show you:

- A firmware flowchart (boot → init → USB setup → GPIF streaming loop).

- A sample C code snippet for handling vendor-specific USB commands (so you can control SSR).

Would you like me to explain that next?