Perfect — now we switch to the next session from your event agenda:

Microcontrollers, Memories and Communication Interfaces for New Space
(Éric Tinlot — Principal Product Marketing Engineer)

We’ll again go step-by-step and convert this PDF into a concept-wise learning report, with:

✅ Fundamentals
✅ Product families
✅ Latest upgrades vs previous generations
✅ Space vs COTS differences
✅ Architecture understanding

All technical points below come from your uploaded session PDF.
Source:

🚀 Step 7 — New Space Electronics Stack (Big System View First)

The first key idea from the deck:

Microchip is not offering just MCUs — but a full space electronics stack.

From the Comprehensive Space Portfolio slide (page 2):

They cover:

- Space MCUs (8-bit → M7 → Rad-Hard)

- Space MPUs

- RT FPGAs & SoC FPGAs

- Communication interfaces (SpaceWire, Ethernet, CAN)

- Memories (SRAM, NOR Flash, NVM)

- Timing & Oscillators

- Power devices

- RF products

This means spacecraft boards can be built with same-vendor interoperable parts.

🧠 Step 7.1 — COTS → Rad-Tolerant → Rad-Hard Strategy (Very Important)

From pages 4–6 diagrams.

🔄 Microchip “Scalable Hardening” Model

They start with:

Industrial / Automotive COTS device

↓

Radiation-Tolerant (RT) version

↓

Radiation Hardened By Design (RHBD) version

Slide states (page 5):

- Same mask set

- Same functionality

- Same development tools

- Same pinout

- Process tuning for:

  - SEL immunity

  - Higher TID

  - SEU characterization

✅ Why This Is a Major Upgrade Approach

Compared to older space chips:

Old model:

Completely separate rad-hard design

Different tools

Different software

Different boards

New model:

COTS → RT → RHBD scaling

Same ecosystem

Faster development

Lower risk

This is a New Space enabler.

🧱 Step 7.2 — Space MCU Family Evolution (From Your Deck)

From pages 9–17 — we can build a clean generation ladder.

🔹 Generation 1 — 8-bit Rad-Tol AVR

From page 9:

Devices:

- ATmegaS64M1

- ATmegaS128

Class:

- ~10 MIPS

- 8-bit AVR core

- CAN / SPI / UART

- Motor control

- Small memory

- TID ~30 krad

- SEL immune to ~62 MeV

Use cases:

- Simple control nodes

- Power control

- Motor control

- Housekeeping

🔹 Generation 2 — Cortex-M0+/M3 Rad-Tol

From pages 10–11:

SAMD21J17RT — M0+

- ~50 DMIPS

- DMA + bus matrix

- 12-bit ADC

- DAC

- SERCOM flexible interfaces

- TID > 50 krad

- SEL up to 78 MeV

SAM3X8ERT — M3

- ~100 DMIPS

- Dual CAN

- Ethernet

- External memory

- Dual-bank Flash/SRAM

- TID 30 krad

🔹 Generation 3 — Cortex-M7 Rad-Tol (High Performance)

From page 12:

SAMV71Q21RT — Major Upgrade

- Cortex-M7 @ 300 MHz

- ~600 DMIPS

- DSP + FPU

- CAN-FD

- Ethernet

- Crypto AES

- Integrity Check Monitor (SHA)

- DMA 24-channel

- ECC caches

- External memory controller

- TID 30 krad

This is a big jump vs M3/M0+.

☢️ Step 7.3 — Rad-Hard M7 SoC — Next Upgrade Tier

From pages 13–15:

🛰️ SAMRH71 — Rad-Hard SoC (Major Platform)

Key specs:

- Cortex-M7 @ 100 MHz

- 200 DMIPS

- ECC everywhere

- Large SRAM + TCM

- SpaceWire ×2

- MIL-STD-1553

- CAN-FD

- Ethernet TSN

- Hardened external memory controller with ECC

- Dual watchdog

- Integrity Check Monitor

- TID > 100 krad

- No SEFI / no Flash SBU/MBU

⚡ Performance Upgrade vs Older SPARC (Slide shows benchmark)

From performance slide (page 15):

Customer algorithm comparison:

| Core | Algo1 | Algo2 |
| --- | --- | --- |
| LEON3-FT | 4.3 ms | 2600 ms |
| SAMRH71 M7 | 1.4 ms | 548 ms |

That is:

3–5× faster vs LEON3-FT on real workloads

Same mW/DMIPS ratio — better efficiency.

🧩 Step 7.4 — New Upgrade — SAMRH707 (Rad-Hard MCU with Analog)

From pages 17–18:

🆕 SAMRH707 — What’s New vs SAMRH71

Differences:

- Smaller package options

- Integrated ADC + DAC (12-bit)

- Lower clock (50 MHz)

- Lower DMIPS (~100+)

- More analog integration

- Still:

  - SpaceWire ×2

  - 1553

  - CAN-FD

  - ECC memories

  - TID > 100 krad

  - QML & ESCC qualified

Slide explicitly markets:

“Rad-Hard Microcontroller with advanced analog functions”

✅ Upgrade Insight

SAMRH71 → compute-heavy node
SAMRH707 → control + analog integrated node

Board designers can remove external ADC/DAC chips.

🧭 Step 7 Summary — MCU Upgrade Ladder

AVR RT → basic control

M0+/M3 RT → modern control

M7 RT → high performance

M7 Rad-Hard → mission critical compute

M7 Rad-Hard + Analog → integrated control nodes

Major upgrades across generations:

- Performance: 10 → 600 DMIPS

- Interfaces: UART → SpaceWire + 1553

- Protection: basic → ECC everywhere

- Security: none → crypto + integrity monitor

- Integration: digital → digital + analog

▶️ Next Step (Step 8)

Next we go into:

Space Communication Interfaces — SpaceWire, Ethernet PHY, CAN-FD RT — device-level understanding + what’s new vs older space links

Say next and we continue.

Great — continuing step-by-step. Now we move to the next major block from your session:

🛰️ Space Communication Interfaces for New Space Systems
(From your uploaded “Microcontrollers, Memories and Communication Interfaces for New Space” deck)

We’ll convert this into a clean engineering module:

✅ Why special space interfaces are needed
✅ SpaceWire vs Ethernet vs CAN in spacecraft
✅ Radiation-tolerant PHY & transceiver products
✅ What’s upgraded vs older generations
✅ How they connect with RT MCUs & FPGAs

All technical points come from your uploaded session PDF.
Source:

🚀 Step 8 — Why Space Communication Interfaces Are Special

Spacecraft communication links are not just “protocol choices” — they are architecture decisions.

They must handle:

- Radiation faults

- Deterministic timing

- Long cable runs inside spacecraft

- EMI sensitivity

- Redundancy

- Fault containment

- Time synchronization

- Mixed criticality traffic

That’s why Microchip portfolio includes rad-tolerant versions of:

- SpaceWire

- Ethernet PHY

- CAN-FD transceivers

- Clock & timing generators

Shown in the communication & interface product slides.

🛰️ Step 8.1 — SpaceWire — Spacecraft Data Backbone

From multiple slides (MCU + processor + interface sections):

SpaceWire appears repeatedly in:

- SAMRH71

- SAMRH707

- AT7910 router

- GNSS SoCs

- Rad-hard processors

🧠 What SpaceWire Is (Concept)

SpaceWire is a space-specific high-speed serial network used inside spacecraft.

Used for:

- Payload data routing

- Instrument interconnect

- Onboard network backbone

- Deterministic packet transfer

✅ Why It’s Preferred in Space

Advantages:

- Deterministic latency

- Packet routing

- Fault containment

- Lightweight protocol

- LVDS physical layer

- Built-in flow control

- Widely standardized in ESA missions

🔧 Product Upgrades Shown in Your Deck

From SAMRH71 / SAMRH707 slides:

Newer MCUs now integrate:

- Dual SpaceWire ports

- RMAP support

- Integrated LVDS transceivers

That’s an upgrade vs older designs that needed:

MCU + external SpaceWire controller + external LVDS

Now → fewer chips → lower SWaP.

🌐 Step 8.2 — Radiation-Tolerant Ethernet PHY — Major Upgrade Area

Your deck shows dedicated RT Ethernet PHY devices.

🔹 VSC8541RT — Rad-Tol Gigabit Ethernet PHY (page 20)

Key features from slide:

- 10/100/1000BASE-T

- RGMII / GMII / MII / RMII MAC interface

- IEEE 1588 timing support

- Sync Ethernet support

- Low EMI line driver

- Host-free configuration (strap pins)

- –55°C to +125°C

- Ceramic + plastic packages

Radiation:

- TID 100 krad

- SEL immune up to 78 MeV

🆕 What’s Upgraded vs Older Space Ethernet

Older space Ethernet often required:

- custom PHY

- lower speeds

- higher power

- less timing precision

Upgrades now:

✅ Gigabit support
✅ IEEE-1588 timing
✅ Lower EMI drivers
✅ Lower power class
✅ RT qualified

This supports:

- onboard high-rate data networks

- TSN (time sensitive networking)

- synchronized payload systems

🔌 Step 8.3 — Multi-Port RT Ethernet PHY — Higher Integration

🔹 VSC8574RT — 4-Port RT PHY (page 21)

From slide:

- 4-port GbE PHY

- Copper + fiber support

- SGMII / QSGMII

- 5 Gbps SERDES

- IEEE 1588v2 high accuracy (~8 ns)

- –55°C to +125°C

Radiation:

- TID 100 krad

- SEL immune to 78 MeV

✅ Upgrade Meaning

Instead of:

4 separate PHY chips

You can use:

1 quad-port RT PHY

Benefits:

- Board area reduction

- Power reduction

- Timing alignment

- Easier sync network design

🚌 Step 8.4 — Rad-Tolerant CAN-FD — Control Bus Layer

🔹 ATA6571RT — RT CAN-FD Transceiver (pages 22–23)

From slides:

- CAN-FD up to 5 Mbit/s

- Low EMI emission

- High ESD robustness

- Functional safety docs (FMEDA)

- Remote wake-up

- Bus transient protection

- –55°C to +125°C

Radiation:

- TID > 50 krad

- SEL immune to 78 MeV

🆕 Upgrade vs Classic Space CAN

Older space CAN:

- Lower speed

- No FD frame extension

- Less diagnostic support

CAN-FD RT adds:

✅ Higher data payload per frame
✅ Faster data phase
✅ Better diagnostics
✅ Better safety documentation

🔧 Integration Upgrade Highlight

Slide shows:

SAMRH707 evaluation board integrates ATA6571RT CAN transceiver

Meaning:

Rad-hard MCU + rad-tol CAN PHY = ready control node stack.

⏱️ Step 8.5 — Space Timing & Clock Interfaces (Newer Focus)

🔹 DSA504RT — RT Clock Generator (page 24)

From slide:

- 6 differential / 12 CMOS outputs

- Up to ~665 MHz outputs

- PCIe Gen1–Gen5 compliant clocks

- 250 fs jitter

- I2C configurable

- Multi-voltage support

Radiation:

- TID > 100 krad

- SEL > 62.5 MeV

✅ Why This Is an Upgrade Area

Modern space systems now include:

- PCIe

- TSN Ethernet

- high-speed SERDES

- SoC FPGAs

These require low-jitter multi-output clocks — older simple crystal drivers are not enough.

🧭 Step 8 Summary — Communication Stack Evolution

From your session material, the upgrade direction is clear:

Old Space Stack

- SpaceWire + basic CAN

- Lower speed Ethernet

- External controllers

- Many discrete chips

New Space Stack (Upgraded)

- Integrated SpaceWire in MCUs

- Gigabit RT Ethernet PHY

- Multi-port PHY

- CAN-FD RT

- RT clock generators

- IEEE-1588 timing

- TSN-ready networks

- Higher integration

All shown across interface product slides.

▶️ Next Step (Step 9)

Next we move to:

Space Memories — RT NOR Flash, Serial Flash, SRAM — architecture role + upgrades vs older space memory

Say next and we continue building your final concept-wise report.

Good 👍 — I’ll continue fast but still structured, so your final report stays concept-wise and technically solid.

We now cover the next block from your session:

💾 Space Memories for New Space Systems
(From your Microchip “MCUs, Memories & Communication Interfaces” deck)

I’ll compress but keep the engineering clarity + upgrade comparisons.

All technical points come from your uploaded PDF.
Source:

💾 Step 9 — Space Memory Types (System View First)

From the memory slides, Microchip provides radiation-tolerant memory families:

Main Space Memory Classes

| Type | Use in Spacecraft |
| --- | --- |
| Parallel NOR Flash | Boot + program storage |
| Serial NOR Flash | Config + code + data |
| SRAM | Fast runtime memory |
| EEPROM / EERAM | Parameters & logs |

Key design drivers:

- Radiation tolerance

- Data retention years

- Write endurance at high temp

- Execute-in-Place (XIP)

- Deterministic read timing

🧱 Step 9.1 — RT Parallel NOR Flash — High Reliability Code Storage

🔹 SST38LF6401RT — 64 Mbit Parallel NOR (page 25)

From slide:

- 64 Mbit organized as 4M × 16

- 3.0–3.6 V

- –55°C to +125°C

- 90 ns read access

- Page read buffer

- Fast erase (~40 ms typical)

- 10k erase cycles @125°C

- 20-year data retention

Radiation:

- TID 50 krad

- SEL immune up to 78 MeV

✅ Where Used

- Boot memory

- RTOS image

- FPGA bitstream storage

- Safe program store

🆕 Upgrade vs Older Space Flash

Upgrades highlighted:

- Faster page read

- Better endurance at high temp

- Plastic + ceramic options

- Rad-tol process tuned from COTS base

⚡ Step 9.2 — RT Serial Quad NOR Flash — High-Speed + Lower Pin Count

🔹 SST26LF064RT — 64 Mbit Serial NOR (page 26)

From slide:

- Quad-I/O SPI (SQI)

- Up to 80 MHz operation

- Execute-In-Place (XIP)

- No SRAM shadow copy required

- 3.0–3.6 V

- –55°C to +125°C

- 10k cycles

- 20-year retention

Radiation:

- TID 50 krad

- SEL immune up to ~62.5 MeV

✅ Major Upgrade Concept — XIP

Older designs:

Flash → copy to SRAM → execute

Now:

Flash → execute directly (XIP)

Benefits:

- Faster boot

- Less SRAM needed

- Lower power

- Simpler software

Important for small rad-tol MCUs.

🧠 Step 9.3 — Memory Protection Upgrades in New MCUs

From SAMRH71 / SAMRH707 / V71RT slides:

New generation space MCUs include:

- ECC on Flash

- ECC on SRAM

- ECC on TCM

- External memory ECC controller

- Integrity Check Monitor (SHA)

This is a big upgrade vs older AVR / M3 class devices.

From SAMRH71 memory block diagram slides.

🔋 Step 9.4 — Power Support for Memory Rails

Memory reliability depends on clean rails.

🔹 MIC69303RT — RT LDO Regulator (page 27)

From slide:

- 1.65–5.5 V input

- Up to 3A output

- Adjustable output

- Soft start

- Low dropout

- –55°C to +125°C

Radiation:

- TID 50 krad

- SEL immune 78 MeV

✅ Why Included in Memory Section

Because:

Flash + SRAM upset rates increase with:

- voltage noise

- droop

- brownout

RT LDOs + supervisors are part of memory reliability chain.

⏱️ Step 9 Fast Summary — Space Memory Upgrades

New Space memory stack improvements:

✅ Rad-tol serial & parallel NOR options
✅ Quad-SPI high throughput
✅ XIP support
✅ High-temp endurance ratings
✅ ECC inside MCUs
✅ External memory ECC controllers
✅ Plastic + ceramic flows
✅ Faster access vs legacy parts

All from your session memory slides.

▶️ Next (Final Block for This Session — Fast Mode)

Next I’ll quickly finish:

Step 10 — System Use Cases + Integration Patterns
(from the last slides: motor control, telemetry, in-flight reprogramming, MCU + FPGA combos)

Say go and I’ll complete the last section and then assemble your full structured report.

Excellent — final fast pass for this session. I’ll finish the last block and then you’ll have a clean, concept-wise understanding of the whole:

MCU + Memory + Communication Interfaces for New Space
(from your Microchip session PDF)

All points below come from the last slides of your uploaded deck.
Source:

🛰️ Step 10 — Space System Integration Use Cases (How These Parts Are Actually Used)

The last slides shift from products → system-level design patterns.

This is important because interviews often ask:

“How would you architect a space embedded system using these devices?”

Microchip shows reference system combinations.

⚙️ 10.1 — Motor Control in Space Systems

From use-case slides (motor control examples):

🚀 Typical Space Motor Applications

- Solar panel positioning

- Antenna pointing

- Valve control

- Camera gimbals

- Micro-positioning stages

- Reaction mechanisms

🧩 Example Stack from Slides

Motor Control Node

- Rad-Hard MCU → control loop

- Motor driver IC → power stage

- RT analog blocks → sensing

- RT CAN / SpaceWire → command link

- Timing IC → clock stability

Slides show:

SAMRH71 + motor control drivers
SAMRH707 motor control demo projects (GitHub + Harmony)

🆕 Upgrade vs Older Approach

Before:

Rad MCU + many external analog + custom drivers

Now:

Rad-hard MCU + integrated ADC/DAC + RT drivers + ready software stack

Software ecosystem is part of the upgrade.

📡 10.2 — Telemetry & Data Handling Node

From system use case slide:

Example Combo Shown

Telemetry node:

- RT FPGA (RTG4 / PolarFire) → data formatting

- RT MCU → control & protocol

- RT Ethernet PHY → downlink network

- RT memory → buffering

Slide explicitly lists:

Telemetry with RTG4 and LX7730

🧠 Architecture Pattern

Sensors → FPGA datapath → MCU control → Ethernet / SpaceWire → OBC

This matches earlier:
control path vs data path split.

🔄 10.3 — In-Flight Reprogramming Architecture

From final system slides:

Highlighted Pattern

In-Flight Reprogramming
SAMRH71 + RT FPGA

How It Works (Concept)

- RT MCU = supervisor

- RT FPGA = reconfigurable logic

- RT NOR Flash = bitstream storage

- RT interface = update channel

Flow:

Ground → upload new image → stored in RT Flash → verified → FPGA reprogrammed in orbit

This is a New Space upgrade capability vs legacy fixed-function payloads.

🧰 10.4 — Software & Toolchain Integration (Hidden but Important Upgrade)

From software ecosystem slides:

Supported stacks shown:

- MPLAB ecosystem

- Harmony framework

- RTOS ports

- Linux (for higher devices)

- IAR / Keil support

- BSP packages

- Evaluation boards

- Reference designs

✅ Why This Is a Real Upgrade

Older space parts:

- custom toolchains

- limited IDE support

- harder onboarding

Now:

- Same dev tools as COTS cousins

- Same IDE families

- Faster bring-up

- Easier hiring/training

🧭 Final Session Summary — MCU + Memory + Interface Track

From your entire uploaded session, the upgrade direction vs previous generations is clear:

🧠 Architecture Evolution

Old Space Electronics

- Custom rad-hard only parts

- Low performance

- Many discrete chips

- Limited interfaces

- Custom software stacks

- Long dev cycles

🚀 New Space Electronics (From This Session)

✅ COTS → RT → RHBD scalable families
✅ AVR → M0 → M3 → M7 → Rad-Hard M7
✅ Integrated SpaceWire + 1553 + CAN-FD
✅ Gigabit RT Ethernet PHY
✅ Quad-SPI XIP flash
✅ ECC everywhere memory
✅ Integrated crypto & integrity monitors
✅ FPGA + MCU cooperative designs
✅ In-flight reprogramming support
✅ Plastic + ceramic qualification paths
✅ Modern toolchains & eval kits

All documented across your session slides.

📘 Next What I’ll Do (Report Assembly Phase)

Now we have covered — step by step — two full sessions:

1️⃣ RT FPGA Update
2️⃣ MCUs + Memories + Interfaces

Next step (no more theory blocks):

I will assemble your final systematic concept-wise report with:

- Topic sections

- Upgrade comparison tables

- Product evolution ladders

- System architecture patterns

- Interview-ready notes

Upload remaining PDFs (Power, Oscillators, Analog, RF, PIC64-HPSC) — I’ll merge them into the same structured report.

Perfect — this is the PIC64-HPSC MPU session from your forum. I’ll keep it fast + structured + concept-wise, and highlight:

✅ What PIC64-HPSC actually is
✅ How it differs from MCUs & FPGAs
✅ Architecture breakdown
✅ Latest upgrades vs previous space processors
✅ Why it matters for future spacecraft (AI + high data rate payloads)

All points come from your uploaded deck.
Source:

🚀 Step 11 — What is PIC64-HPSC? (Big Picture First)

PIC64-HPSC = High-Performance Spaceflight Computing MPU

This is not a microcontroller.
This is not just an FPGA.

It is a:

🧠 Rad-hard / rad-tolerant 64-bit multi-core space processor platform
built for:

- onboard AI

- autonomous missions

- high-bandwidth payload compute

- secure edge processing

From opening slides:
Focus areas include AI/ML, autonomy, cybersecurity, high-bandwidth interfaces, and fault tolerance.

🧠 Step 11.1 — Why HPSC Exists (Problem It Solves)

From “Driving Forces for HPSC” slide:

Space Compute Demand Is Exploding

Drivers:

- AI / ML onboard

- Autonomous navigation

- Advanced sensors & cameras

- Moon–Mars missions

- Edge intelligence

- Cybersecurity threats

- Faster dev cycles

Old space CPUs (SPARC, LEON, simple rad MCUs):

❌ Too slow
❌ No AI acceleration
❌ Limited bandwidth I/O
❌ Weak virtualization

HPSC is the next compute tier.

🧩 Step 11.2 — PIC64-HPSC Core Architecture

From architecture slide (page 4):

🔹 Compute Core

- 64-bit RISC-V

- SiFive X280 / X288 class

- Vector processing

- AI acceleration

- Virtualization support

🔹 Multi-Core Structure

From silicon diagram slide:

- 8 × RISC-V application cores

- Vector + real-time support

- Multi-level cache

- Lockstep options

🔹 Integrated Major Blocks

Also integrated:

- TSN Ethernet switch (huge)

- PCIe / CXL

- RDMA engines

- Secure enclave

- System controller CPU

- Secure controller CPU

This is SoC-class space compute, not MCU-class.

🔐 Step 11.3 — Defense-Grade Security (Major Upgrade)

From security slides:

PIC64-HPSC includes:

- Hardware root of trust

- Secure boot

- Tamper detection

- Secure enclave

- Post-quantum crypto support:

  - ML-KEM

  - ML-DSA

- Side-channel & DPA resistant design

This is beyond typical space MCU crypto.

Older space processors → classic crypto only
HPSC → post-quantum ready

☢️ Step 11.4 — Fault Tolerance Features (Next-Gen)

From feature highlights:

Built-In Fault Tolerance

- DCLS (dual core lockstep)

- Split-mode operation

- WorldGuard isolation

- Hardware partitioning

- Error containment regions

Supports:

- mixed-critical workloads

- isolation between safety & non-safety tasks

🌐 Step 11.5 — Massive Connectivity (Huge Upgrade Area)

From architecture & networking slides:

Integrated High-Bandwidth Fabric

Built-in:

- 240G TSN Ethernet switch

- 10GbE

- PCIe Gen3

- CXL 2.0

- RDMA / RoCEv2

- Deterministic networking

🆕 Why This Is a Big Deal

Old architecture:

CPU  +  external switch  +  PCIe bridge  +  FPGA fabric

HPSC:

Compute + TSN switch + PCIe + RDMA → on one chip

Lower:

- latency

- board area

- power

- integration risk

🤖 Step 11.6 — AI / ML Acceleration Built-In

From AI slide:

HPSC includes:

- RISC-V vector extensions

- Dedicated vector pipeline

- AI data transform engine

- Framework support:

  - TensorFlow Lite

  - PyTorch

  - ONNX

✅ Upgrade vs Old Space Design

Before:

Space CPU + separate AI accelerator + FPGA glue

Now:

AI-capable vector CPU on-chip

Still expandable via PCIe/CXL if needed.

🧠 Step 11.7 — Mixed-Criticality on One Processor

From mixed-criticality slide:

Problem

Traditionally:

- safety workload → one CPU

- payload compute → another CPU

Increases:

- SWaP

- complexity

- cost

HPSC Solution

Supports:

- hardware partitioning

- virtualization

- AMP + SMP modes

- cache isolation

- lockstep zones

Run together but isolated:

- mission-critical control

- high-performance payload compute

🛰️ Step 11.8 — Product Variants — Orbit Class Targeting

From positioning slide:

Two Main Variants

🔹 PIC64-HPSC1100-RT

- New space / commercial

- ~50 krad class

- LEO/MEO

- AEC-Q100 style

🔹 PIC64-HPSC1000-RH

- Rad-hard

- 200 krad class

- GEO / deep space

- NASA class missions

- QML-Y flow

Common footprint + software stack.

🛠️ Step 11.9 — Software Stack (Modern)

From ecosystem slides:

Supports:

- Linux

- RTEMS

- VxWorks

- Hypervisors

- AI toolchains

- LLVM / GCC

- TSN stacks

- RDMA stacks

This is server-class style ecosystem, unusual for space processors.

📈 Step 11.10 — Technology Node Upgrade

From milestone slide:

Fabrication:

- GlobalFoundries 12nm

- Hundreds of millions of gates

- Hundreds of millions SRAM bits

- Large FCBGA package

This is a huge node shrink vs older rad-hard CPUs (150nm / 180nm class).

✅ Step 11 Fast Summary — PIC64-HPSC Upgrade Impact

Compared to previous space processors (SPARC / LEON / classic rad MCUs):

🚀 Major Upgrades

✅ 64-bit RISC-V vector compute
✅ Built-in AI acceleration
✅ Multi-core + virtualization
✅ Integrated TSN switch
✅ PCIe + CXL + RDMA
✅ Post-quantum security
✅ Mixed-criticality isolation
✅ Rad-hard + rad-tol variants
✅ Modern OS ecosystem
✅ Advanced node (12nm)

📘 Next Step

Remaining sessions you mentioned:

- Space Power & Discrete

- Oscillators & Timing

- Mixed-Signal & Analog

- RF Products

Upload those PDFs — I’ll finish the last fast modules and then produce your final full systematic concept report across all sessions.

Good — this is your Space-Grade Mixed Signal & Analog Products session. I’ll keep it fast + structured + upgrade-focused, same style as previous sections.

This block is very important because it explains how Microchip reduces:

✅ Board size
✅ Component count
✅ Failure points
✅ Power risk
in spacecraft electronics using integrated rad-hard analog/mixed-signal ICs.

All technical points below come from your uploaded deck.
Source:

🔬 Step 13 — Why Mixed-Signal & Analog ICs Matter in Space Systems

From opening slides:

These parts target:

- Motor control & position sensing

- Telemetry acquisition

- Spacecraft health monitoring

- Fault detection

- Power protection & distribution

- Redundant power buses

- SEL protection for COTS loads

Key theme from slide:

Reduce BoM → reduce size & weight → increase reliability

🧱 Step 13.1 — Product Families Covered

From product table slide (page 3):

Production Space Analog ICs

| Device | Function |
| --- | --- |
| AAHS298B | 8-channel high-side driver |
| LX7710 | 8-pair diode array |
| LX7720 | Motor control system manager |
| LX7730 | Telemetry system manager |

New / Sampling (Upgrades)

| Device | Function | Status |
| --- | --- | --- |
| LX7712 | Programmable current limit switch | Sampling |
| LX7714 | Quad resettable e-Fuse | Sampling |
| LX7722 | Dual smart power switch | Sampling 2026 |

⚡ Step 13.2 — High-Side Drivers & Diode Arrays (Power Bus Building Blocks)

🔹 AAHS298B — 8-Channel Source Driver

From feature slide:

- 80 V breakdown

- 700 mA per channel

- Fully isolated channels

- Thermal shutdown

- TTL/5V/12V logic compatible

- Ceramic SOIC

Radiation:

- 100 krad TID

- SEL immune to 87 MeV

- QML Q & V

🔹 LX7710 — 8-Pair Diode Array

- 125 V breakdown

- 1 A per diode

- Redundant diode paths

- Inductive kickback discharge

- Ceramic SOIC

Radiation similar class.

Slides show redundant power bus switch system diagram using AAHS298B + LX7710.

🧠 System Insight

These are used to build:

Redundant spacecraft power buses

channel switching

fault isolation

kickback protection

Upgrade vs discrete diodes + drivers = fewer parts + qualified behavior.

🧠 Step 13.3 — Space System Manager ICs — Major Integration Upgrade

This is one of the most important upgrades in this session.

🧩 Concept: Space System Manager (SSM)

From slide definition:

Special mixed-signal IC used with MCU or FPGA
Adds analog + power + sensing capability
Replaces up to 24 discrete ICs

⚙️ LX7720 — Motor Control System Manager

From pages 9–12 slides:

Integrated:

- 4 half-bridge MOSFET drivers

- Current sensing

- Resolver / LVDT interface

- PWM driver

- Position sensing inputs

- Fault detection

- Protection logic

Radiation:

- 100 krad TID

- SEL immune

- QML Q/V + plastic sub-QML

Flight heritage slides show use in:

- Artemis Orion

- Mars Sample Return

- ISS systems

- ESA missions

🆕 Upgrade Highlights

New dev ecosystem:

- LX7720-DB dev board

- Works with:

  - RTG4

  - PolarFire

  - SAMRH71

  - SAMRH707

- FOC motor control demo code

- Resolver + sensorless examples

This is a toolchain & integration upgrade, not just silicon.

📡 LX7730 — Telemetry System Manager

From telemetry slides:

Integrated:

- 64-channel analog MUX

- 12-bit ADC

- Precision reference

- Current source

- Comparators

- DAC

- Parallel / SPI interface

Radiation:

- 100 krad TID

- SEL immune 87 MeV

- QML Q/V + plastic

Slide shows:

50–75% board space reduction
Increased reliability

🧠 Upgrade vs Old Telemetry Boards

Before:

MUX + ADC + ref + DAC + comparators + drivers

= many chips

Now:

LX7730 = single qualified IC

🔥 Step 13.4 — New Power Protection ICs — Big New Space Upgrade

These are newer generation protection parts.

🔹 LX7712 — Programmable Current Limiting Power Switch

From page 16:

- 18–126 V switching

- Internal PMOS + diode

- Programmable UVLO

- Slew rate control

- Parallelable

- Over-temp protection

- ESA standards compliant

Use:

- Power distribution

- Load protection

- Bus fault isolation

🔹 LX7714 — Quad Resettable e-Fuse

From page 17:

- 4 independent switches

- 14–46 V

- Resettable protection

- Hiccup retry mode (4096 retries)

- Fault timer logic

- Thermal protection

🔹 LX7722 — Smart Dual Power Switch (NEW)

From page 20–21:

- 1A / 2A integrated switches

- I2C programmable limits

- Current monitoring

- Cold sparing mode

- Back-to-back isolation

- Very small DFN package

- SEL latch-up protection use case shown for DDR4 banks

🧠 Upgrade Trend

Old approach:

Fuse + sense + comparator + logic

New approach:

Single rad-tol smart switch IC

📦 Step 13.5 — Plastic Space Packages — New Space Upgrade

Slides show plastic package options for:

- LX7720MLF

- LX7730MLF

- LX7714 plastic concepts

Benefits:

- Smaller footprint

- Lower cost

- Faster availability

- LEO / New Space friendly

- Proven package heritage

Matches earlier RT Mil-Plastic strategy seen in FPGA session.

✅ Step 13 Fast Summary — Mixed-Signal & Analog Upgrades

From this session:

🚀 Major Upgrade Themes

✅ System Manager ICs replace 20+ discretes
✅ Integrated motor control + telemetry analog
✅ Smart power switches with SEL protection
✅ Resettable e-fuses for spacecraft buses
✅ Rad-hard + plastic variants
✅ Dev boards for FPGA + MCU pairing
✅ Strong flight heritage + new parts sampling
✅ BoM reduction → reliability increase

⏭️ Next Final Module

Remaining session you mentioned:

RF Products for Space Applications

Upload that last PDF — then I’ll generate your final full systematic concept-wise master report across ALL sessions.

Excellent — this is your final remaining session:

📡 RF Products for Space Applications (Oscar Mansilla)

I’ll keep this fast, structured, and upgrade-focused like the previous ones, and extract:

✅ RF product categories
✅ Technology platforms (GaAs, GaN, SAW, PIN, MMIC)
✅ Space screening & packaging levels
✅ New upgrades vs older RF space parts
✅ Complete RF signal chain view

All technical points come from your uploaded RF session deck.
Source:

📡 Step 14 — RF in Space Systems (System Role First)

From the opening slides:

RF devices support:

- SATCOM links

- GNSS receivers

- Radar & phased arrays

- EW / ECM receivers

- Telemetry & guidance

- Scientific sensing

- RF switching & attenuation

- Power amplification (CW + pulsed)

Slide shows 40+ years RF space heritage and many supported missions.

🧱 Step 14.1 — RF Technology Platforms (Radiation Angle)

From page 2 slide:

Microchip RF space devices are built using:

🔹 Process Technologies

- GaAs MMIC

- GaN on SiC

- Silicon PIN & Schottky

- SAW filter processes

Important statement from slide:

These processes are inherently radiation immune (GaAs, GaN-on-SiC)

Meaning:
They are naturally more tolerant than bulk CMOS RF.

Also supports:

- MIL-PRF-38534 screening

- ESCC3502 screening

- Class K MMIC flows

- Hermetic packaging options

🧩 Step 14.2 — RF Product Families (Portfolio Map)

From RFDS solutions table (page 7):

🔹 Wideband MMICs

Products:

- LNAs

- Power amplifiers

- Switches

- Attenuators

- Prescalers

- Phase frequency detectors

Coverage:

- DC → 70 GHz

Key upgrade theme:

✅ Wideband + programmable + high linearity

🔹 RF Diodes & Diode Modules

Types:

- PIN diodes

- Schottky

- Varactors

- GaAs PIN

- Gunn diodes

- Beam-lead diodes

- MMSM monolithic diodes

Use:

- RF switching

- limiters

- TR switches

- duplexers

- protection

🔹 GaN & Si Power Discretes / Pallets

- UHF → Ku band

- Avionics

- Radar

- AESA arrays

- kW-class RF power

Upgrade theme:

✅ GaN = high efficiency + SWaP reduction

📡 Step 14.3 — Phased Array RF Chain (Architecture Slide)

From phased-array diagram slide (page 8):

Typical chain includes:

- GaAs/GaN switches

- Drivers & PAs

- Attenuators

- Limiters

- LNAs with high NF performance

Trend bullets on slide:

- Wider bandwidth

- Higher frequency

- Higher power

- More packaged devices

- GaN-driven growth

This shows shift from discrete → semi-integrated RF tiles.

🚀 Step 14.4 — Wideband High-Linearity MMIC Amplifiers (Upgrade Class)

From MMIC amplifier slide (page 9):

Process:

- 0.15µm PHEMT GaAs

- Fully passivated

- Space-qualified EPPL flow

Performance:

- OIP3 up to 43 dBm

- P1dB up to 1 W

- DC–26 GHz class parts

- Die + QFN packages

Example parts listed in table (page 9):

- MMA051AA

- MMA151AA

- MMA155AA

- MMA052AA

Upgrade theme:

✅ Higher linearity
✅ Wider band
✅ Qualified process flow

🛡️ Step 14.5 — High Power Handling LNAs (Protection Upgrade)

From page 10 slide:

🔹 High Power Handling LNA

- MMIC handles >32 dBm CW

- With PIN limiter → >10 W protection

- Hermetic AlN package option (new)

Meaning:

Receiver front-end survival improved vs older fragile LNAs.

🔀 Step 14.6 — PIN Diode Families (Switching & Protection)

From PIN diode product slide (page 11):

Families:

- GC42xx — high-speed switching

- GC44xx — power PIN to 100W

- GC46xx — large signal to 1 kW

- GC48xx/49xx — beam lead

- GaAs PIN families

Frequency use:

- 0.1–40 GHz switching

- Radar & EW

- TR switches

- SDR radios

Upgrade theme:

✅ Very high power classes
✅ Custom screening
✅ Space class S & K flows

🧩 Step 14.7 — MMSM Monolithic RF Diodes — Packaging Upgrade

From MMSM slide (page 12):

🔹 MMSM = Monolithic Microwave Surface Mount

Key innovation:

Device + package integrated at wafer level

Benefits:

- Replaces die

- Surface-mount ready

- Minimal parasitic change

- Up to 100W CW

- MSL1 rating

This is a manufacturing & assembly upgrade.

📶 Step 14.8 — Varactors for Smart Antennas

From antenna varactor slide (page 14):

Features:

- DC–100 GHz tuning

- Very high Q (~30k)

- Ultra low capacitance (0.03 pF)

- Tiny 0201 size

- GaAs material

- AEC-Q101 qualified

Used in:

- SATCOM

- phased arrays

- 5G/6G antennas

Upgrade theme:

✅ Electronically tunable antennas
✅ Compact phased array tuning

🆕 Step 14.9 — New MMIC Releases (High Frequency Upgrade)

From new MMIC slide (page 15–16):

New parts:

- MMA119AA — Q-band 47–53 GHz

- MMA127AA / MMA137AA — dual-band Satcom:

  - 27.5–31 GHz

  - 37.5–42.5 GHz

Ku-band block shows:

- Class-K qualified parts

- GaN PA MMICs

- 8W–20W class devices

Upgrade:

✅ Move into mmWave Satcom bands
✅ Higher frequency qualified MMICs

🛰️ Step 14.10 — SAW RF Filters — Space GNSS Upgrade

From SAW filter slides (pages 19–21):

🔹 High-Reliability SAW Filters

Types:

- Precision SAW

- Low-loss SAW

- IF SAW

- GNSS SAW

Performance:

- Very sharp skirts

- Shape factor ≈ 1.03

- Up to 2.7 GHz

- Ceramic & metal packages

GNSS space solution slide shows:

- L1/E1 and L5/E5a filters

- Gull-wing package for thermal cycling reliability

- ECSS compliant package design

Upgrade theme:

✅ Space-grade SAW with improved package reliability

🧭 Step 14.11 — Full RF Signal Chain Integration (Key Slide)

From RF signal chain diagram (page 22):

Shows complete chain:

- Limiter PIN diodes (0–40 GHz)

- Wideband MMIC switches (to 26 GHz)

- SAW filters (to 2.7 GHz)

- Wideband GaN LNAs & PAs (to 65 GHz)

- Prescalers (to 40 GHz)

- Frequency detectors

- GaN HEMT PAs

Meaning:

Microchip now offers end-to-end RF signal chain, not isolated parts.

✅ Step 14 Fast Summary — RF Upgrade Trends

From your RF session:

🚀 Major Upgrades vs Older RF Space Parts

✅ GaN-on-SiC high power RF
✅ Wideband DC–70 GHz MMIC portfolio
✅ mmWave Satcom MMIC releases
✅ High-power tolerant LNAs
✅ MMSM wafer-level packaged RF diodes
✅ Smart antenna varactors
✅ Space-grade SAW GNSS filters
✅ Full RF signal-chain vendor coverage
✅ Plastic + hermetic screening paths

📘 Next Message — Final Deliverable

You’ve now uploaded and we’ve processed all session PDFs.

Next message I will deliver:

🧾 Your Full Systematic Concept-Wise Master Report

covering all sessions with:

- Concept map

- Product evolution ladders

- Upgrade vs previous generation tables

- System architecture patterns

- Interview-ready notes

- Embedded/space career relevance

Say generate report and I’ll assemble it cleanly.

Perfect 👍
Now I’ll generate your final consolidated concept-wise report from all the Microchip Space Forum sessions you attended.

I will follow your instructions:

✅ Each main topic explained in one place
✅ Start from basics → then advanced → then product upgrades
✅ Easy, beginner-friendly but technically correct
✅ System understanding focus (not interview notes)
✅ Add practical usefulness instead of interview prep
✅ Clean structure so you can revise anytime

This is your Microchip Space Systems — Concept Master Report.

All content is derived from your uploaded session PDFs.

🚀 Microchip Space Systems — Concept Master Report

1️⃣ Space Electronics — Big Picture Architecture

Modern spacecraft electronics is built as layered blocks:

Sensors / Payloads

↓

Analog + Mixed Signal Front End

↓

MCU / MPU / FPGA Processing

↓

Memory + Storage

↓

Communication Interfaces

↓

RF Frontend + Power Amplifiers

↓

Timing & Oscillators

↓

Power Distribution & Protection

Microchip’s space portfolio covers every layer — that was the core theme across all sessions.

Upgrade trend vs old space design:

- Old: many discrete parts, custom chips

- New: integrated rad-tolerant platforms + reusable ecosystems

2️⃣ Radiation Fundamentals (Applies to All Devices)

☢️ What Space Radiation Does

Three main effects:

SEU — Single Event Upset

- Bit flip in memory or logic

- Temporary

- Fixed by:

  - TMR (triple modular redundancy)

  - ECC memory

  - scrubbing

SEL — Single Event Latchup

- Internal short circuit

- Dangerous

- Needs:

  - rad-hard process

  - current limiting

  - smart power switches

TID — Total Ionizing Dose

- Long-term degradation

- Measured in krad

- Handled by:

  - rad processes

  - SOI tech

  - special layout

Upgrade trend:
New devices include built-in ECC + TMR + fault monitors — not just rad silicon.

3️⃣ Radiation-Tolerant FPGAs (RT FPGA Session)

🧠 What an RT FPGA Does

Used for:

- High-speed data paths

- DSP pipelines

- Packet processing

- Sensor fusion

- AI acceleration blocks

- Protocol engines

Two roles:

Control path → safety logic
Data path → heavy compute

🧱 FPGA Technology Evolution

Generation path:

RTSX / RTAX → Antifuse (one-time, very immune)

RTG4 → Flash + built-in TMR

RT PolarFire → Flash + soft-TMR + high density

RT PolarFire SoC → FPGA + RISC-V CPUs

🚀 RT PolarFire — Major Upgrades

Compared to RTG4:

- ~3–6× more logic

- SERDES up to 12.7 Gbps

- ~2× power efficiency

- Soft-TMR via tools

- EDAC memories

- In-flight reprogramming

- Strong security (PUF, secure boot)

- Plastic + ceramic options

System benefit:

👉 One FPGA can now replace multiple legacy FPGAs.

🧩 RT PolarFire SoC — New Architecture Class

Adds:

- Multi-core RISC-V CPU

- Large L2 cache

- Security enclave

- FPGA fabric

Meaning:

CPU + FPGA → one rad-tolerant chip

Reduces board size and power.

4️⃣ Space Microcontrollers (MCU Session)

🧠 What Space MCUs Do

Used for:

- System control

- Telemetry handling

- Motor control

- Fault management

- Protocol stacks

- Supervisory tasks

🔄 MCU Evolution Ladder

8-bit AVR RT → basic control

Cortex-M0/M3 RT → modern control

Cortex-M7 RT → high performance

Rad-Hard M7 → mission critical

Rad-Hard M7 + Analog → integrated nodes

🚀 Modern Space MCU Upgrades

New M7 rad-hard class (SAMRH71 / SAMRH707):

- ECC everywhere

- SpaceWire inside MCU

- MIL-STD-1553

- CAN-FD

- Ethernet TSN

- Crypto + SHA integrity monitor

- Memory scrubbing

- Dual watchdog

- Fault monitors

Upgrade vs older SPARC / LEON class:

👉 Much higher performance per watt
👉 Modern toolchains
👉 Better integration

5️⃣ High-Performance Space Processors — PIC64-HPSC

🧠 What This Class Is

This is space MPU / AI processor class — above MCU.

Used for:

- AI onboard

- autonomy

- image processing

- high-bandwidth payloads

- edge computing

🚀 PIC64-HPSC Upgrades

- 64-bit RISC-V vector cores

- Multi-core

- AI acceleration

- TSN Ethernet switch on-chip

- PCIe + CXL + RDMA

- Post-quantum crypto

- Virtualization

- Mixed-criticality isolation

- 12 nm node

Old space CPUs → control
HPSC → compute + AI

6️⃣ Space Memories

💾 Memory Roles

- Boot storage

- Program storage

- FPGA bitstreams

- Data logs

- Parameter storage

Types:

- Parallel NOR

- Serial Quad NOR

- SRAM

- EEPROM

🚀 Memory Upgrades

New RT memory features:

- XIP (execute in place)

- High-temp endurance

- Quad-SPI speed

- Rad-tol serial flash

- ECC in MCUs

- External memory ECC controllers

System benefit:

👉 Faster boot
👉 Less SRAM needed
👉 Higher reliability

7️⃣ Space Communication Interfaces

🛰️ Main Space Links

SpaceWire

- Space network backbone

- Deterministic packets

- Now integrated inside rad MCUs

RT Ethernet PHY

- Gigabit

- IEEE-1588 timing

- Multi-port PHYs

- TSN ready

CAN-FD RT

- Control bus

- Higher payload than classic CAN

- Rad-tol transceivers

🚀 Interface Upgrades

Old:
external controllers + PHYs

New:
integrated SpaceWire + rad PHY + TSN timing

Result:

👉 fewer chips
👉 better sync
👉 smaller boards

8️⃣ Oscillators & Timing

⏱️ Why Timing Is Critical

Clocks drive:

- CPUs

- FPGAs

- SERDES

- Ethernet

- GNSS

- TSN

Clock drift = system failure risk.

🧱 Oscillator Classes

XO → basic
TCXO → temp compensated
OCXO → oven stabilized
EMXO → miniature oven
CSAC → atomic clock

🚀 Timing Upgrades

- 300 krad oscillator platforms

- SOI clock technology

- Compact LVDS FPGA clocks

- RT PolarFire clock guides

- Small EMXO modules

- L-series for New Space

- Space CSAC adoption

Benefit:

👉 Better stability
👉 Smaller size
👉 Lower SWaP

9️⃣ Mixed-Signal & Analog Space ICs

🔬 What They Replace

Instead of many discretes:

- ADC

- MUX

- comparators

- drivers

- references

- switches

Now → single System Manager IC

🚀 Key Upgrades

System Manager ICs

Motor manager (LX7720)
Telemetry manager (LX7730)

Replace 20+ parts.

Smart Power Protection

- Resettable e-fuses

- Smart switches

- Programmable current limits

- SEL protection switches

Benefit:

👉 Huge BoM reduction
👉 Higher reliability
👉 Easier certification

🔟 RF Space Products

📡 RF Chain Coverage

- LNAs

- PAs

- MMIC amplifiers

- RF switches

- PIN diodes

- Varactors

- SAW filters

- GaN power devices

🚀 RF Upgrades

- GaN-on-SiC high power

- DC–70 GHz MMICs

- mmWave Satcom parts

- High-power LNAs

- MMSM wafer-level RF diodes

- Smart antenna varactors

- Space GNSS SAW filters

- Full RF chain coverage

Benefit:

👉 Higher frequency
👉 Higher power
👉 Smaller RF modules

✅ Final Practical Takeaways (Most Useful for You)

Since you want usefulness over interview prep — here is what matters most for your embedded/space learning path:

🧠 What You Should Now Clearly Understand

You now understand:

- Why space parts are different from COTS

- How radiation changes design rules

- Why ECC/TMR/SEL protection is everywhere

- Why FPGA + MCU + MPU roles differ

- How modern space systems reduce chip count

- How timing & power are as critical as compute

- How New Space shifts toward integration + cost control

🛠️ What Skills Connect Directly to This Domain

Focus your upskilling on:

- C / C++ embedded

- RTOS

- MCU + peripheral drivers

- Communication protocols

- Digital design basics

- FPGA fundamentals

- Memory + bus architecture

- Fault-tolerant design thinking

If you want, next I can convert this full report into:

✅ Clean PDF notes
✅ Concept revision sheets
✅ Space embedded learning roadmap
✅ Project ideas aligned with this stack.

Got it 👍 — you don’t want rule lists.
You want foundation-first explanation:

👉 What each thing is
👉 Why it exists
👉 What problem it solves
👉 When we use it
👉 What happens if we don’t use it

I’ll re-explain the RT PolarFire FPGA Design Tips & Tricks session again — but from absolute basics → practical space design thinking — simple and grounded.

No product hype — only engineering reasoning.

All concepts come from your uploaded RT PolarFire FPGA Design Tips & Tricks deck.
Source:

🛠️ RT PolarFire FPGA — Design Tips Explained From Basics

Think of this whole session as answering one question:

“How do we make an FPGA design survive radiation faults in space?”

Because space problem is not speed — it is random faults.

Let’s build from ground up.

1️⃣ First — What Is Special About Space FPGA Design?

On Earth FPGA design goal:

Correct logic + timing + power

In space FPGA design goal:

Correct logic + timing + power + fault survival

Because in space:

- bits flip randomly (SEU)

- controllers can glitch

- config registers can corrupt

- high-speed blocks can misconfigure

So design must be:

self-protecting + self-recovering

Everything in this “tips & tricks” session is about that.

2️⃣ System Controller Suspend Mode (SCSM)

🧠 First — What is “System Controller” inside FPGA?

Modern FPGAs have an internal mini-controller that handles:

- configuration

- security

- programming

- device services

It is like a hidden microcontroller inside the FPGA.

❗ Problem in Space

That controller is also digital logic → radiation can upset it.

If it gets upset while running:

- wrong internal command

- wrong config action

- possible device disturbance

✅ Solution — Suspend It

Suspend Mode = keep that internal controller OFF during mission runtime

Why?

Because:

Inactive logic cannot fail

You only wake it when needed:

- update

- verification

- service operation

🛰️ When Used

Used in:

- long mission runtime

- stable deployed systems

- payload FPGAs

Not needed during:

- lab debug

- development stage

3️⃣ Reset & Restart Logic — Why It Is Discussed

🧠 Basic Truth

Radiation can cause:

- unexpected reset

- partial reset

- glitch reset

If your FPGA logic assumes:

“reset happens only once at power-on”

👉 that assumption is wrong in space.

❗ What Happens Without Reset Design

- FSM stuck mid-state

- counters half-cleared

- interfaces misaligned

- outputs unsafe

✅ Why Special Reset Blocks Are Used

Reset monitor IP helps:

- detect restart

- reinitialize safely

- bring system to known state

🛰️ When Used

Always — in any space FPGA with control logic.

4️⃣ TMR — Triple Modular Redundancy (From First Principles)

🧠 What Is SEU Problem?

Radiation hit → flip-flop changes value.

Example:

state = IDLE

bit flip → state = RUN

System jumps to wrong behavior.

✅ Basic Idea of TMR

Instead of 1 register → use 3 copies:

A = IDLE

B = IDLE

C = IDLE

Add voter:

majority wins

If one flips:

IDLE, IDLE, RUN → voter = IDLE

System continues correctly.

❗ Why Not TMR Everything?

Because:

- uses 3× area

- reduces speed

- increases routing

🛰️ When We Use TMR

Use for:

- state machines

- mode control

- safety signals

- configuration registers

Avoid for:

- large datapaths

- DSP pipelines

- bulk data paths

5️⃣ Synthesized TMR — Why Tool-Based?

Older RT FPGAs had built-in TMR flip-flops.

RT PolarFire does not.

So tools generate TMR automatically.

Why tool-based is good:

- consistent triplication

- placement separation

- voter insertion

- fewer human mistakes

6️⃣ CDC — Clock Domain Crossing (From Basics)

🧠 What Is CDC?

Two clock domains:

logic A → clock 50 MHz

logic B → clock 80 MHz

Signal crosses between them.

❗ Problem

Receiving flip-flop may sample during transition → metastability.

Output becomes:

- unpredictable

- delayed

- wrong

Radiation increases timing noise → CDC risk worse.

✅ Basic CDC Fix

Use:

two flip-flop synchronizer

Why two?

First may go metastable
Second stabilizes result

🛰️ When Used

Anytime signals cross clocks:

- control signals

- interrupts

- status flags

7️⃣ Why CDC + TMR Discussion Exists

Because designers ask:

Should synchronizers also be TMR?

Answer:

- Usually NO

- Only for critical CDC

- Because synchronizer behavior is special

So tool avoids auto-TMR of CDC unless designer forces it.

8️⃣ Transceiver SEU Recovery — From Basics

🧠 What Is Transceiver Block?

High-speed serial block:

- SERDES

- protocol layer

- link training

- equalization

Configured by registers.

❗ Problem

Registers are volatile → SEU can change config.

Then:

- link drops

- wrong speed

- framing error

- data loss

❌ Bad Design Pattern

configure once at boot → never check again

Works on Earth — fails in space.

✅ Correct Space Pattern

Add logic that:

- monitors link health

- checks status flags

- re-initializes block if needed

Self-healing design.

🛰️ When Used

Always — if using SERDES / transceivers in space.

9️⃣ Register Lock Bits — Why Mentioned

🧠 Basic Idea

Some config registers never need to change after setup.

Example:

mode = LVDS

never changes

❗ SEU Risk

If SEU flips it:

LVDS → CMOS

Interface breaks.

✅ Lock Bit Solution

Lock register → cannot be changed → SEU write blocked.

🛰️ When Used

Use for:

- static configuration

- fixed protocol settings

- constant modes

Not for runtime control registers.

🔚 Final Plain-English Summary

RT PolarFire design tips are about one principle:

Do not assume your FPGA will run perfectly forever — design it to detect, tolerate, and recover faults.

So we:

- turn off risky internal controllers when not needed

- duplicate critical logic (TMR)

- stabilize clock crossings (CDC)

- design reset recovery

- monitor high-speed blocks

- lock important registers

- allow re-initialization

If you want, next I can convert this into a one-page “Space FPGA Survival Design Guide” you can reuse in future projects.

Good catch 👍 — yes, this last session is important and very practical:

RT PolarFire® FPGA Design Tips & Tricks

This one is not about products — it is about how to design correctly with RT PolarFire in radiation environments. I’ll explain it fast, simple, and practical — basics → what to do → why → common mistakes.

All points come from your uploaded deck.
Source:

🛠️ RT PolarFire FPGA — Design Tips & Tricks (Practical Guide)

Think of this session as:

✅ “How to design your FPGA logic so it survives radiation faults”

Not theory — real design rules.

Main topics covered in the slides:

- RT vs Non-RT PolarFire differences

- System Controller Suspend Mode (SCSM)

- Synthesized TMR usage

- Clock Domain Crossing (CDC) handling

- SEU-recoverable transceiver design

- Register lock for SEU protection

We go one by one.

1️⃣ RT vs Non-RT PolarFire — What Changes for Designer

RT PolarFire is based on normal PolarFire — but enhanced for space:

Key enhancements (from comparison tables):

- Better SEL immunity

- ECC added to internal NVM

- Improved in-flight programming checks

- System Controller Suspend Mode support

- Extra verification during updates

- Space screening levels (QML-Q, planned QML-V)

Design meaning:
Do not treat RT device exactly like terrestrial one — enable the RT features.

2️⃣ ⭐ System Controller Suspend Mode (SCSM) — Very Important

🧠 What is System Controller?

Inside PolarFire there is an internal system controller that handles:

- configuration

- services

- security

- update

- system tasks

But controller logic itself can be upset by radiation.

🛡️ What SCSM Does

System Controller Suspend Mode = keep controller in reset during normal operation

Slides say:

- System controller held in reset after power-up

- Mitigates controller SEU errors

- Enabled via configuration bit

✅ Design Rule

During normal mission operation:

Keep SCSM enabled

Only wake controller when needed for:

- system services

- verification

- update

- digest check

Exit methods shown:

- JTAG TRSTB

- SC_WAKE signal (SoC devices)

3️⃣ Reset & Initialization Handling — Must Be Designed

Slides emphasize:

You must design fabric logic to handle:

- unexpected device reset

- restart detection

- reset sequencing

Use provided IP:

- PF_INIT_MONITOR

- CORERESET_PF

- PFSOC_INIT_MONITOR

Also:

- latch controller outputs before SCSM entry

- use internal 160 MHz RC oscillator for monitor logic

Meaning:
Reset handling must be intentional — not default.

4️⃣ ⭐ Synthesized TMR — How RT PolarFire Does TMR

Important difference vs RTG4:

RTG4 → built-in TMR flip-flops
RT PolarFire → no built-in TMR FF

So you must use:

Synthesized TMR

🧠 Two TMR Styles

Local TMR

Triplicate flip-flops + voter

Distributed TMR

Triplicate full logic cone + vote output

🔧 How To Enable (Tool Method)

Slides show Synplify directive:

syn_radhardlevel = "tmr"

Tool creates:

reg_tmr1

reg_tmr2

reg_tmr3

voter block

Placed spatially separated in fabric.

⚠️ Tradeoff

Slides state:

- ~30–40% timing slowdown

- Use only for critical control logic

Not for whole design blindly.

5️⃣ TMR Practical Rules (Very Useful)

From tips slides:

Do:

✅ TMR state machines
✅ TMR control registers
✅ TMR safety logic

Avoid:

❌ Blind TMR of RAM pipelines
❌ Hard-IP internal registers
❌ High-speed datapaths unless required

Also:

- use syn_preserve on key registers

- disable aggressive optimization that breaks triplication

- check synthesis & layout logs for TMR report

6️⃣ ⭐ Clock Domain Crossing (CDC) — Radiation + Metastability Risk

CDC = signal crossing between two clocks.

Problem:

- metastability

- timing uncertainty

- worse under radiation upset

🧠 Tool Support (Synplify CDC Analysis)

Slides show:

Tool can:

- detect CDC synchronizers

- tag them

- preserve them

- place FFs close together

- reduce metastability window

Requires:

- async clocks declared in SDC

- virtual clocks for async inputs

✅ CDC Design Rules

Use:

2-FF synchronizer

Enable:

- CDC detection option

- review CDC report file

- keep synchronizer FFs packed

Attribute:

cdc_synchronizer = 1

7️⃣ TMR + CDC Together — Special Case

Slides show case study:

Some designers:

- never TMR CDC

- some always TMR CDC

Tool default:

Synplify will NOT TMR detected CDC automatically

If you want TMR CDC:

- manually code triplicated synchronizer

- add voter

- preserve naming

- disable auto TMR

- tag safe CDC

This is advanced — only for critical crossings.

8️⃣ ⭐ SEU-Recoverable Transceiver Design — Very Important

Key statement from slides:

RT PolarFire transceivers are not inherently SEU mitigated

Because:

- built from standard ASIC cells

- config registers are volatile

SEU can corrupt them.

🛠️ Required Solution

Do NOT rely only on power-up init.

Instead:

Build fabric-based re-initialization engine

Using:

- CoreABC controller

- TMR controller logic

- stored init data

- error detection monitors

🔍 What To Monitor

Re-init transceiver if:

- status signals drop

- PCS errors

- data errors

- link faults

Slides show fabric re-init architecture.

Meaning:
Transceiver = monitor + recover, not fire-and-forget.

9️⃣ Register Lock Bits — Simple but Powerful Trick

Hard IP blocks have config registers.

Some registers support:

Flash-based lock bits

If locked:

- SEU cannot change value

- writes disabled

✅ Best Use

Lock registers that:

- stay at default

- never change at runtime

Do NOT lock registers needing runtime writes.

Tool generates:

_init_config_lock_bits.txt

You change lock value → feed back to flow.

Result:

👉 Reduced SEU surface area.

✅ Final Simple Summary — RT PolarFire Design Rules

If you remember only this — you’re safe:

🛡️ Radiation-Safe Design Checklist

✅ Enable SCSM
✅ Design proper reset & restart handling
✅ Use TMR for critical control logic
✅ Use ECC RAM + scrubbing
✅ Enable CDC detection & proper synchronizers
✅ Monitor PLL lock & status signals
✅ Build transceiver re-init logic
✅ Lock unused config registers
✅ Review radiation reports vs mission profile
✅ Check synthesis & layout logs for TMR/CDC results

If you want, next I can merge this Tips & Tricks section into your master report and produce a final structured PDF-style document for your study and revision.