Perfect 👍 — we’ll go step by step, concept by concept, exactly like a structured technical class.

We’ll start with your first uploaded session file:

Radiation-Tolerant (RT) FPGA Update — Microchip (Minh Nguyen)
I’ll convert this into a clean learning module — not just slides summary — but real understanding.

All points below are derived from your uploaded PDF.
Source reference:

🚀 Step 1 — Why RT FPGAs Are Becoming Critical in Space (Big Picture First)

Before device names — understand the problem space.

From the presentation early slides:

🌍 Space Industry Is Changing

Key drivers:

- Launch cost is decreasing

- LEO constellations increasing

- More satellites → more onboard processing

- Less downlink bandwidth → more onboard compute

- Compute → heat → power problem

From slides:

“Massive compute generates massive heat”
“Power dissipation is the #1 problem in the data path”

🧠 Why Not Just Use CPUs/GPUs in Space?

Because space systems need:

- Deterministic timing

- Low power per operation

- Radiation tolerance

- Reconfigurable logic

- Hardware acceleration

- No configuration corruption

👉 That’s where Radiation-Tolerant FPGAs fit best.

🛰️ Step 2 — Control Path vs Data Path (Very Important Concept)

This is one of the most important architecture ideas from the talk.

🔹 Control Path = Mission Brain Logic

Examples:

- Command decoding

- State machines

- Redundancy voting

- Safety logic

- Power sequencing

- System supervision

Needs:

- Absolute reliability

- Configuration SEU immunity

- Proven heritage

Slides say:

Control Path → configuration SEU immunity valuable

Used devices historically:

- RTSX-SU

- RTAX

- RT ProASIC3

- RTG4

🔹 Data Path = Payload Compute Logic

Examples:

- Image processing

- Signal processing

- Compression

- Encryption

- AI/ML inference

- Sensor pipelines

Needs:

- High throughput

- High logic density

- SERDES speed

- Power efficiency

Slides say:

Data Path → 2× power efficiency valuable

This is where RT PolarFire comes in.

⚡ Step 3 — Microchip RT FPGA Strategy (Core Message of Talk)

From the vision slide:

Microchip FPGA strategy pillars:

1️⃣ Power efficiency (2×)
2️⃣ Zero configuration upsets
3️⃣ Military-grade security

Source slide: Vision & Strategy

This is positioning vs competitors who focus mainly on raw performance.

In space → watts matter more than TOPS.

🔥 Step 4 — Why Power Is More Important Than Raw Performance in Space

From thermal slides:

🌡️ Space Thermal Reality

- No air convection

- Only:

  - radiation

  - conduction

- Heat removal is hard

- Thermal systems = heavy + costly

Slides state:

- Excess heat → performance degradation

- Thermal control → cost increase

- Reliability decreases with temperature

📉 Reliability vs Temperature (Critical Insight)

Slide shows:

- Failure rate doubles every ~10°C rise

- PolarFire runs cooler than competitor FPGAs

- Lower FIT rate (failures in time)

Meaning:

👉 Lower power FPGA = longer mission reliability

🔐 Step 5 — Security in Space FPGAs (New Emphasis Area)

This is newer vs older FPGA generations.

RT PolarFire security features:

- PUF-based key storage

- Anti-tamper

- Side-channel resistance

- Secure boot

- Bitstream encryption

- Device certificates

- Supply chain verification

From slide: “Cradle to Grave Security”

This is a big upgrade vs older RTAX / RTSX families.

🧱 Step 6 — FPGA Technology Types (Must Understand for Interviews)

From portfolio table slides:

Microchip RT FPGA families use different base tech:

| Family | Technology | Reprogrammable | SEU Behavior |
| --- | --- | --- | --- |
| RTSX-SU | Antifuse | ❌ One-time | Very immune |
| RTAX | Antifuse | ❌ One-time | Very immune |
| RT ProASIC3 | Flash | ✅ | Good |
| RTG4 | Flash | ✅ | Built-in TMR |
| RT PolarFire | SONOS Flash | ✅ | Soft TMR + EDAC |

Source: portfolio table

🧠 Key Concept — Why Antifuse Was Used Historically

Older space FPGAs used antifuse because:

- One-time programmable

- No configuration memory → no SEU bit flips

- Very stable in radiation

But downside:

❌ Not reprogrammable
❌ Lower density
❌ Slower dev cycles

Newer generation → radiation-tolerant flash FPGA + mitigation

✅ Step 7 — Major Upgrade: RT PolarFire Generation

From roadmap slides:

RT PolarFire brings:

- Much higher logic elements (~481K LE)

- High-speed SERDES (12.7 Gbps)

- Lower power vs competitors

- Reprogrammable

- Soft TMR + EDAC protection

- QML-Q → path to QML-V

Slide highlights:

3×–6× more resources
50% faster vs prior RT families

🧭 Next Step — What We Cover Next

Next step we will go deep into:

▶️ Step 2 Module:

RT PolarFire vs RTG4 vs RTAX — Generational Upgrade Comparison

We will cover:

- Architecture difference

- Radiation handling methods

- Configuration upset strategy

- TMR vs EDAC

- Soft vs hard mitigation

- In-flight programming capability

- Qualification classes (QML-Q / V / Y / JEDEC)

Say “next” — we continue step-by-step.

Excellent — now we go into the most important technical foundation behind everything you heard in the RT FPGA session:

This step explains:

✅ What radiation actually does to electronics
✅ SEU vs SEL vs TID — clearly separated
✅ How RT FPGAs are designed to survive
✅ What exactly RT PolarFire improved vs older generations

All technical points are based on your uploaded Radiation-Tolerant FPGA Update deck.
Source:

☢️ Step 3 — Radiation Effects in Space Electronics (Engineer-Level Understanding)

Space radiation is not one single thing. It creates different failure modes.

You must separate them clearly:

| Radiation Effect | Type | Damage Style | Reversible? | Design Response |
| --- | --- | --- | --- | --- |
| SEU | Bit flip | Logic/memory upset | ✅ Yes | TMR / EDAC |
| SEL | Latchup | Parasitic short | ❌ No (dangerous) | Process + guard design |
| TID | Dose | Parameter drift | ❌ No | Rad-hard process |

We go one by one.

⚡ 3.1 — SEU (Single Event Upset)

🧠 What It Is

A charged particle hits:

- flip-flop

- SRAM bit

- configuration memory bit

It deposits charge → bit flips.

Example:

Stored bit = 1

Radiation hit → becomes 0

No physical damage — but logic result becomes wrong.

📍 Where SEUs Matter Most in FPGAs

- Configuration bits

- User flip-flops

- Block RAM

- State machines

- Counters

- FIFOs

Older SRAM FPGAs are very sensitive here.

🛡️ How RT PolarFire Mitigates SEU

From SEE summary slide:

✅ Flip-Flops Protection

- Tool-generated soft TMR

- Libero synthesis inserts triplication

- Cluster separation rules

“Protected flip-flops have synthesized soft-TMR with Libero-driven cluster separation”

✅ SRAM Protection

- Built-in EDAC (SECDED)

- Error Detect + Correct

From slide table:

| Memory | Protection |
| --- | --- |
| LSRAM | SECDED |
| uSRAM | EDAC |

Upset rate drops dramatically:

Unprotected FF: ~1.3e-7
Protected FF: ~1.2e-11

🎯 Engineering Insight

SEU is expected in space.

Design goal is not “prevent all SEU” — it is:

Detect + correct + vote + recover

🔥 3.2 — SEL (Single Event Latchup) — Dangerous One

🧠 What SEL Is

Radiation triggers parasitic transistor structure:

CMOS contains hidden PNPN path → becomes short circuit.

Result:

- Sudden high current

- Thermal runaway

- Device destruction if not limited

This is not a bit flip — this is a current fault.

⚠️ Why SEL Is Serious

SEU → wrong answer
SEL → dead chip

Requires:

- power cycle

- current limiting

- latchup immune process

🛡️ RT PolarFire SEL Improvements

From SEL summary slide:

ZT variants improved vs older T versions.

Example from table:

| Device | SEL LET threshold |
| --- | --- |
| RTPF500T | lower |
| RTPF500ZT | higher (improved) |

Slide notes:

RTPF500T obsolete → RTPF500ZT replacement with better SEL radiation

⚙️ Board-Level Requirement (Important!)

Slide note:

No destructive SEL unless board decoupling capacitor requirement is not followed

Meaning:

Hardware design still matters.

Rad device ≠ careless board allowed.

☀️ 3.3 — TID (Total Ionizing Dose)

🧠 What TID Is

Long-term radiation exposure causes:

- Threshold voltage drift

- Leakage increase

- Timing change

- Analog degradation

Measured in:

krad(Si)

📈 RT FPGA TID Ratings (From Portfolio Slide)

| Family | TID |
| --- | --- |
| RT PolarFire | ~100 krad |
| RTG4 | ~100 krad |
| RTAX | up to 300 krad |

🎯 Practical Meaning

Device is guaranteed to function correctly up to that accumulated dose.

Mission orbit determines required TID rating.

🔄 3.4 — In-Flight Programming Under Radiation

This is a major modern capability.

Older antifuse FPGAs:

❌ cannot reprogram

RT PolarFire:

✅ supports in-flight programming

From slide:

Supported up to 100 krad TID and 60 MeV LET
Probability of success > 99%
Retry if first attempt fails

🧠 Why This Is Big

Enables:

- On-orbit updates

- Algorithm upgrade

- Patch bugs

- Mission adaptation

- AI model update

This is critical for New Space missions.

🧮 3.5 — Upset Rate Modeling (Orbit Dependent)

Slides show upset rate examples:

Metrics used:

Upsets per day

Years between upsets

GEO vs LEO difference

Example MSS upset stats table shows:

- watchdog timeout upset every ~15–120 years depending core

- reset events extremely rare

🎯 Engineering Lesson

Radiation mitigation is:

- statistical

- probabilistic

- modeled per orbit

- not binary safe/unsafe

🧱 3.6 — Configuration SEU Immunity — Why Microchip Pushes It

From earlier strategy slides:

Zero configuration upsets
Control path values SEU immunity

Microchip flash/SONOS FPGA config memory:

- far more stable than SRAM config FPGAs

- no continuous configuration scrubbing required

- lower system complexity

✅ Step 3 Summary — Radiation Handling Stack

RT PolarFire protection stack:

Layered defense:

1️⃣ Process-level rad-tolerant silicon
2️⃣ Flash/SONOS config memory stability
3️⃣ Soft-TMR for flip-flops
4️⃣ EDAC for RAM
5️⃣ SEL-improved ZT silicon
6️⃣ Tool-assisted mitigation (Libero)
7️⃣ Board decoupling rules
8️⃣ Statistical orbit modeling

▶️ Next Step (Step 4)

Next we move to:

RT PolarFire SoC architecture — FPGA + RISC-V CPUs — and how this changes space system design compared to MCU + FPGA split architectures

Say next and we continue.

Perfect — now we move to a major architectural upgrade discussed in your RT FPGA session:

This step is about:

🚀 RT PolarFire SoC — FPGA + multi-core RISC-V processor in one radiation-tolerant device

This is where space hardware design is clearly moving:
from MCU + FPGA boards → single SoC FPGA platforms.

All technical points come from your uploaded RT FPGA Update deck.
Source:

🧠 Step 4 — What Is RT PolarFire SoC (Concept First)

Traditional space board design:

Rad MCU  +  RT FPGA  +  External memory  +  Security IC

RT PolarFire SoC integrates:

RT FPGA Fabric + Multi-core CPU + Cache + Security + Memory controllers

From the slide:

“Industry’s only built-in processor subsystem”
Quad-core 64-bit RISC-V + 1 monitor core
Linux + RTOS support

🧩 Step 4.1 — CPU Subsystem Inside RT PolarFire SoC

🔹 Core Structure

RT PolarFire SoC contains:

- 4 × 64-bit RISC-V application cores

- 1 × monitor / management core

- Asymmetric multiprocessing support

Slide text:

Quad-core 64-bit RISC-V processors + 1 monitor core

🔹 Why RISC-V Is Important for Space

Engineering reasons (not marketing):

✅ Open ISA → long mission lifetime safety
✅ No vendor lock ISA risk
✅ Easier security audit
✅ Deterministic control possible
✅ Custom extensions possible
✅ Radiation testable core RTL lineage

For space programs, toolchain independence matters.

⚙️ Step 4.2 — FPGA Fabric + CPU — Division of Work

This is how designers actually use SoC FPGAs.

🧮 FPGA Fabric Handles

- High-speed datapath

- DSP pipelines

- Packet processing

- Sensor fusion

- Compression

- Crypto accelerators

- Protocol engines

- Custom interfaces

🧠 CPU Subsystem Handles

- System control

- Mission software

- Scheduling

- Telemetry

- Fault management

- Health monitoring

- File systems

- Linux services (if used)

🎯 Architecture Pattern

CPU = control plane

FPGA = data plane

This exact separation was emphasized in your session slides
(control path vs data path concept).

💾 Step 4.3 — Cache & Memory Architecture Upgrade

From the SoC slide:

🔹 Large L2 Cache

- ~2 MB L2 cache

- Shared across cores

- Reduces external memory access

- Lowers power

- Improves determinism

Slide text:

Highly flexible 2 MB L2 cache

🧠 Why This Matters in Space

External memory:

- higher SEU risk

- higher power

- more latency

- more board complexity

Large on-chip cache:

👉 fewer external transactions
👉 fewer upset exposure points
👉 better timing determinism

🔐 Step 4.4 — Built-In Security Subsystem

RT PolarFire SoC includes:

- Secure boot

- Root of trust

- Bitstream encryption

- Key storage

- Anti-tamper

- Device certificates

- Supply chain verification

From “Cradle to Grave Security” slide.

🛰️ Why SoC-Level Security Is a Big Upgrade

Older designs required:

- external secure element

- board-level crypto IC

- custom boot chain

Now:

✅ security is on-chip
✅ fewer attack surfaces
✅ fewer parts
✅ easier certification

🔄 Step 4.5 — Mixed Criticality Systems (Important Concept)

Slide mentions:

SWaP-C advantages for mixed criticality systems

🧠 Mixed Criticality = Different Safety Levels in One Device

Example:

| Function | Criticality |
| --- | --- |
| Flight control | High |
| Telemetry | Medium |
| Payload processing | Medium |
| AI inference | Low |

SoC enables:

- separation

- isolation

- privilege control

- monitor core supervision

⚡ Step 4.6 — Power Efficiency vs Competitors

From benchmark slide:

RT PolarFire SoC shows:

- Much higher CoreMarks/Watt

- Lower thermal rise

- Lower FIT rate due to temperature

Slide example:

PolarFire SoC hits high CoreMarks before competitors reach similar performance — due to power efficiency.

☢️ Step 4.7 — Radiation Results for SoC CPU Subsystem

Slides include MSS radiation tables:

Metrics shown:

- Upset rate per day

- Years between upsets

- Watchdog timeout upset modeling

- Reset probability extremely low

Example values shown (GEO case):

- watchdog upset every ~15–35 years typical

- reset events extremely rare

🔧 Step 4.8 — Toolchain & Design Flow Impact

RT PolarFire SoC uses:

- Libero SoC toolchain

- Soft-TMR insertion

- Radiation mitigation flow

- Package migration support (plastic ↔ ceramic)

Slide shows design migration flow steps inside Libero.

🧱 Step 4 Summary — What RT PolarFire SoC Changes

Compared to older space architectures:

✅ Before

- Rad MCU + RT FPGA separate

- External security IC

- More board parts

- More power

- More SEU surface

✅ Now

RT PolarFire SoC:

- FPGA + multi-core CPU

- Large cache

- Built-in security

- Rad-tol qualified

- In-flight programmable

- Lower SWaP-C

- Mixed criticality capable

▶️ Next Step (Step 5)

Next we go into:

QML-Q vs QML-V vs JEDEC vs RT Mil-Plastic — qualification levels, screening flows, and how to choose device grade for mission class

This is extremely important for aerospace interviews.

Say next and we continue.

Good — now we move into a topic that is very important for aerospace & space electronics interviews and real mission design decisions:

This step explains:

🧪 How space FPGAs are qualified
🏷️ What QML-Q, QML-V, QML-Y, JEDEC mean
🧱 Ceramic vs Plastic packages
💰 Cost vs reliability tradeoffs
🚀 When to choose which grade

All details come from your uploaded RT FPGA Update presentation.
Source:

We go clean and systematic.

🧪 Step 5 — Space FPGA Qualification Levels (Mission Reliability Classes)

Space components are not all tested the same way.

They are sold under qualification classes — each class defines:

- screening level

- lot testing

- radiation characterization

- traceability

- burn-in stress

- failure analysis depth

From your slide: “Solutions for Every Space Mission — From COTS to QML”

🏷️ 5.1 — Main Qualification Classes (Microchip RT FPGA)

🔵 QML-V — Highest Space Reliability Class

Used for:

- Human spaceflight

- Deep space missions

- Flagship science missions

- Critical national security payloads

From table slide:

Characteristics:

- Hermetic ceramic package

- Most stringent screening

- Lot-by-lot qualification

- Burn-in testing

- Radiation lot data

- Full traceability

- DPA (destructive physical analysis)

Slide row shows:

NASA Class 1 Hermetic Ceramic — QML-V

🟢 QML-Q — High Reliability Space Class

Used for:

- Most GEO missions

- Many defense payloads

- Proven flight programs

Characteristics:

- Ceramic package

- Strong screening

- Generic group testing

- Radiation characterized

- Less stringent than QML-V

From slide table:

QML-Q — Advanced Hermetic Ceramic

🟡 QML-Y — Plastic Space Class

This is newer — supports New Space.

Characteristics:

- Non-hermetic plastic

- NASA Class 1 plastic flow

- Screening performed

- Lower cost vs ceramic

- Still controlled flow

From qualification table slide.

⚪ JEDEC — Industrial/Hi-Rel Plastic Flow

Used for:

- LEO constellations

- Cost-sensitive missions

- Fast deployment programs

- Technology demonstrators

From RT Mil-Plastic slides:

Qualification: JEDEC
Screening: RT Mil-Plastic flow

🧱 5.2 — Hermetic Ceramic vs Plastic — Real Difference

This is not just packaging — it’s mission philosophy.

🧊 Hermetic Ceramic Package

Properties:

- Metal/ceramic sealed

- Moisture proof

- Outgassing controlled

- Long lifetime stability

- High temp tolerance

Slides show:

- Kovar lid

- Hermetic seal

- Space-grade lid

Best for:

- Long missions

- Deep space

- High radiation

- Critical missions

🧴 Plastic (RT Mil-Plastic)

From your slides:

Silicon identical to QML RT FPGA
Package is plastic BGA
JEDEC qualified

Important insight:

👉 Same die — different package + screening flow

⚠️ Tradeoffs

| Ceramic | Plastic |
| --- | --- |
| Best sealing | Not hermetic |
| Highest reliability | Slightly higher risk |
| Highest cost | Much cheaper |
| Longest lead time | Faster |
| Deep space ready | Mostly LEO/new space |

💰 5.3 — Cost & Lead Time — Big New Space Shift

From cost slide:

RT Mil-Plastic:

- 5–10% of QML FPGA cost

- MOQ = 1

- Lead time ~16+ weeks

🧠 Why This Is Big

Traditional space FPGA:

$50k–$200k class parts (example range)

RT Mil-Plastic:

~10% class

Enables:

- constellation scaling

- rapid iteration

- smaller companies entering space

🔄 5.4 — Plastic ↔ Ceramic Design Migration Flow

Slides show Libero flow:

You can migrate design between:

- plastic package

- ceramic package

Steps shown:

1️⃣ Backup Libero project
2️⃣ Ensure place & route complete
3️⃣ Change device package (FC → CG)
4️⃣ Incremental layout reuse
5️⃣ Re-compile & route

🎯 Meaning

You can:

- prototype on plastic

- qualify on ceramic

- reuse design largely intact

That’s a major development speed upgrade.

🧬 5.5 — Screening Flow Levels (From Table Slide)

Slide lists screening flows:

| Flow | Use |
| --- | --- |
| V | QML-V |
| Q | QML-Q |
| Y | QML-Y |
| M | New Space plastic |
| PROTO | prototype |

Each adds:

- more burn-in

- more temp cycling

- more lot analysis

- more destructive sampling

🚀 5.6 — Why Microchip Emphasizes This Portfolio Spread

From strategy slides:

They want to cover:

COTS → Mil Plastic → QML-Q → QML-V

So one silicon platform supports:

- prototype

- new space

- defense

- deep space

This reduces:

- redesign risk

- requalification burden

- toolchain change

✅ Step 5 Summary — Qualification Decision Logic

Choose QML-V when:

- deep space

- crewed missions

- flagship science

- zero-risk tolerance

Choose QML-Q when:

- GEO missions

- defense payloads

- long heritage programs

Choose QML-Y / RT Mil-Plastic when:

- LEO constellations

- cost constrained

- faster schedules

- redundancy by numbers

▶️ Next Step (Step 6)

Next we go into:

RT PolarFire portfolio deep dive — device families (RTPF500ZT, SoC variants, RTG4 comparison, roadmap, and what exactly was upgraded vs previous generation)

Say next and we continue.

Excellent — now we go into the device-level layer of your RT FPGA session.

Up to now we covered:

- Why RT FPGAs are needed

- Radiation mechanisms

- RT PolarFire SoC concept

- Qualification classes

Now we answer:

🔍 What exactly are the RT PolarFire family devices?
🔍 What changed vs previous generation (RTG4 / RTAX)?
🔍 Which parts are current, which are replaced, what is roadmap?

All details below come from your uploaded RT FPGA Update deck.
Source:

We’ll keep this structured like a product engineering map.

🧱 Step 6 — RT FPGA Portfolio Map (Current Generation)

From the portfolio table slide, Microchip RT FPGA lineup now groups into:

🔵 Legacy / Proven Control-Path Families

- RTSX-SU (antifuse)

- RTAX / RTAX-DSP (antifuse)

- RT ProASIC3 (flash)

- RTG4 (flash + built-in TMR)

🟢 New Generation — Data + Control Path

- RT PolarFire FPGA

- RT PolarFire SoC

These are the main upgrade focus of your session.

🚀 Step 6.1 — RT PolarFire FPGA — Flagship Device

🧩 Main Device Highlighted

RTPF500ZT — Current Primary Device

From highlights slide:

- Qualified to QML-Q

- Flight units shipping

- Improved SEL radiation

- Better in-orbit programming

- Plastic + ceramic options

- MOQ = 1

- Path toward QML-V

Slide states clearly:

RTPF500T is obsolete
RTPF500ZT is the replacement

⚠️ Important Upgrade Note

Old → New Replacement

| Old | Status | New |
| --- | --- | --- |
| RTPF500T | ❌ End of Life | RTPF500ZT |

Reason for replacement:

- Better SEL threshold

- Better on-orbit programming robustness

- Improved radiation margins

This is exactly the kind of “latest upgrade vs previous product” detail you asked to capture.

📊 Step 6.2 — RT PolarFire FPGA — Capability Class

From portfolio table:

🔹 RTPF500ZT Class Capabilities

Approx class values from slide:

- ~481K logic elements

- ~33Mbit memory

- Up to 24 SERDES lanes

- Up to 12.7 Gbps SERDES

- ~1500 I/O pins (package dependent)

- Soft-TMR supported

- EDAC memory blocks

🧠 Practical Meaning

This is not just “bigger FPGA”.

This enables:

- Full payload datapath processing

- Multi-channel DSP

- Radar / SAR processing

- High-rate comms framing

- Crypto engines

- AI inference accelerators

In older RTG4 → many of these required multi-FPGA boards.

🧠 Step 6.3 — RT PolarFire SoC Variants (CPU + FPGA)

From roadmap slides:

🔹 RT PolarFire SoC Devices in Plan

Listed devices:

- RTPFS460ZT

- RTPFS160ZT

Status from slide:

- Sampling now

- Qualification planned:

  - JEDEC

  - QML-Q

  - QML-V

  - QML-Y (for smaller device)

🔧 SoC Adds vs FPGA-only

Compared to RTPF500ZT FPGA:

RT PolarFire SoC adds:

- 5-core RISC-V CPU subsystem

- Monitor core

- L2 cache

- Linux + RTOS support

- On-chip processor subsystem

- Mixed criticality support

Same FPGA fabric class — but now compute integrated.

⚙️ Step 6.4 — RTG4 vs RT PolarFire — Direct Upgrade Comparison

Let’s do a clean engineering comparison.

🧱 RTG4 (Previous Generation Flagship)

From portfolio slide:

- ~150K logic elements

- Built-in TMR fabric

- ~3.125 Gbps SERDES

- Flash FPGA

- Very strong heritage

- QML-Q / QML-V available

Used widely in:

- NASA missions

- ESA missions

- GEO payloads

Slide shows large flight heritage list.

🚀 RT PolarFire — Upgrade Over RTG4

| Dimension | RTG4 | RT PolarFire |
| --- | --- | --- |
| Logic size | ~150K | ~481K |
| SERDES | 3.125 Gbps | 12.7 Gbps |
| TMR | Built-in | Soft-TMR (tool) |
| Power efficiency | Good | ~2× better |
| Security | Basic | Advanced secure boot + PUF |
| SoC option | ❌ | ✅ available |
| Mil-plastic | Limited | Strong offering |

All upgrade claims shown across roadmap & strategy slides.

🔐 Step 6.5 — Security Upgrade vs Older Families

RT PolarFire adds security stack not present in older RTAX / RTG4 class:

From security slide:

- PUF key storage

- Side-channel resistance

- Anti-tamper

- Secure boot

- Bitstream encryption

- Supply chain certificates

This is a generation jump, not a minor revision.

📦 Step 6.6 — Package Options — Ceramic + Plastic Parallel

From packaging comparison slide:

For same device family:

You can get:

- CG = Ceramic hermetic

- FC(G) = Plastic RT Mil-plastic

Same die → different package + screening.

Slide explicitly shows package cross-mapping tables.

🛰️ Step 6.7 — Flight Heritage — Why It Matters

Slides list missions already using:

RTG4 — Strong Heritage

- Landsat-9

- Artemis-1

- JUICE

- DART

- Europa Clipper

- NavIC

- Many others

RT PolarFire — Emerging Heritage

- SDA Tranche missions

- SICRAL-3

- Galileo G2

- Lunar rover programs

🗺️ Step 6.8 — Roadmap Direction (Next Gen RT FPGA)

Roadmap slide states:

Next Generation RT FPGA & RT SoC
3×–6× more resources
Details under NDA

Meaning:

Trend is continuing toward:

- larger fabric

- more SERDES

- more compute

- SoC integration

- better power efficiency

✅ Step 6 Summary — Product Upgrade Snapshot

Old → New Direction

RTAX → antifuse → max SEU immunity
RTG4 → flash + built-in TMR
RT PolarFire → large + fast + efficient + secure
RT PolarFire SoC → FPGA + CPU + cache + security
RT Mil-Plastic → cost & schedule revolution

▶️ Next Step (Step 7)

Next we should move to the next session topic from your event agenda:

👉 Microcontrollers, Memories & Communication Interfaces for New Space
(using the rad-tol MCU PDFs you uploaded earlier — SAMRH71, SAMRH707, SAMD21RT, etc.)

Say next and we switch from FPGA → Space MCUs layer.