The provided document is the data sheet for Micron 4Gb, 8Gb, and 16Gb SLC NAND Flash Memory (MT29F series). It defines the electrical, logical, and timing characteristics of the core storage component that resides within the Solid State Recorder (SSR) system you described.

The PDF is intrinsically linked to the function of your entire system by dictating the exact technical requirements the other components (FPGA, PC, custom protocols) must meet to store and retrieve data reliably.

Here is how the NAND Flash data sheet links to the system components mentioned in your query:

1. Link to Flash NAND and Other Memory

The data sheet precisely defines the flash nand component itself:

- Technology and Organization: The document specifies that the devices use Single-Level Cell (SLC) technology. It details the memory organization, including the page size (e.g., x8 organization has 2112 bytes: 2048 main bytes + 64 spare bytes) and block size (64 pages, or 128K + 4K bytes).

- Performance: It lists critical performance metrics, such as the maximum read page time ($t_R$) of 25 µs (or $t_{R_ECC}$ of 70 µs with internal ECC enabled) and the program page time ($t_{PROG}$) of 200 µs (typical).

- Error Management: The NAND supports internal 4-bit ECC, and requires system software to implement bad block management since blocks may be invalid when shipped from the factory. The first block (address 00h) is guaranteed to be valid with ECC.

2. Link to FPGA (Controller Logic)

The FPGA acts as the bridge's low-level controller, and the PDF dictates the logic the FPGA must contain:

- Asynchronous Interface: The NAND uses an asynchronous data interface for I/O operations, with data, commands, and addresses multiplexed onto the same I/O pins (I/Ox).

- Control Signals: The FPGA must generate the precise timing pulses for the control signals defined in the source: CE# (Chip Enable), CLE (Command Latch Enable), ALE (Address Latch Enable), WE# (Write Enable), and RE# (Read Enable). The R/B# (Ready/Busy#) signal, which indicates target array activity, would be monitored by the FPGA.

- Timing Modes: The FPGA's electrical interface implementation must adhere to specific timing constraints, such as the minimum $t_{RC}$ (READ cycle time) of 20ns (3.3V) or 25ns (1.8V), and $t_{WC}$ (WRITE cycle time). This timing is configurable via Timing mode features (Feature Address 01h).

- Command Set Implementation: The FPGA implements the complex flow control defined by the ONFI NAND Flash Protocol. This includes sequences for:

  - Initialization: Issuing RESET (FFh) as the first command after power-on.

  - Read/Program: Executing multi-cycle command sequences like READ PAGE (00h-30h), which requires the 00h command, five address cycles, and the 30h command.

  - Advanced Features: Utilizing features like Two-Plane Operations to improve throughput, which requires specific command sequencing and adherence to Two-Plane Addressing rules (e.g., the plane select bit BA[6] must be different for each address).

3. Link to PC (Host System) and Protocol Components (32-bit Command, 18-byte Packet)

The NAND data sheet defines the fundamental data unit and status mechanisms that govern the high-level system protocol:

- Data Translation: The 32-bit commands or 18-byte packets used in the high-level SSR system protocol must ultimately translate into the NAND's atomic operations: an 8-bit command, up to five 8-bit address cycles, and page-based data transfer (e.g., 2112 bytes). The FPGA's logic is responsible for this conversion.

- System Status: The system's ability to report success or failure back to the PC depends directly on the NAND's status register. The status register bits report crucial information such as Write protect status, RDY (Ready/Busy), ARDY (Array Ready), and FAIL (Program/Erase/Read error). The FPGA/controller must poll these registers, often using READ STATUS (70h) or READ STATUS ENHANCED (78h), to confirm whether the low-level memory operation satisfied the host's 18-byte packet request.

4. Link to 1553B

The source documentation does not contain any information related to the 1553b protocol, which is a specialized military standard bus. In the context of your SSR system, the FPGA would serve as the critical interface, converting the protocol and data packets received from the 1553b bus into the specific asynchronous command sequences required to operate the Micron NAND Flash.

The Micron NAND Flash memory devices (MT29F series) utilize a highly multiplexed asynchronous bus for communication, managed by various input and output control signals.

1. Inputs and Outputs of the NAND Flash

The flash memory device primarily communicates using a bidirectional I/O bus, controlled by several dedicated input/output signals.

| Signal | Type | Description |
| --- | --- | --- |
| I/O[7:0] (x8) | Bidirectional I/O | Transfers address, data, and command information. |
| I/O[15:0] (x16) | Bidirectional I/O | Used on x16 devices to transfer address, data, and command information. |
| CE# (Chip Enable) | Input | Enables or disables one or more die (LUNs) in a target. |
| CLE (Command Latch Enable) | Input | Loads a command from I/O[7:0] into the command register. |
| ALE (Address Latch Enable) | Input | Loads an address from I/O[7:0] into the address register. |
| WE# (Write Enable) | Input | Transfers commands, addresses, and serial data from the host system to the NAND Flash. |
| RE# (Read Enable) | Input | Transfers serial data from the NAND Flash to the host system. |
| WP# (Write Protect) | Input | Enables or disables array PROGRAM and ERASE operations. |
| LOCK | Input | Enables the BLOCK LOCK function when held HIGH during power-up (available only on the 1.8V device). |
| R/B# (Ready/Busy) | Output | An open-drain, active-low output that provides a hardware method of detecting target array activity. |
| VCC/VSS | Supply | Core power supply and ground connections. |

2. Components Linked To and Internal Architecture

The NAND Flash is an internal component of a larger system, linking to various elements typically residing in a controlling chip or host environment:

- System Controller / Host System: This external logic (like an FPGA or microprocessor) drives the control signals (CE#, CLE, ALE, WE#, RE#, WP#) and reads/writes data over the I/Ox pins.

- System Controller Interrupt: The R/B# signal is an open-drain output that is typically connected to an interrupt pin on the system controller to provide a hardware method of indicating when the target is ready or busy.

- Internal Control Logic: Within the device, signals received at the I/O control circuits are used by the control logic circuits to generate internal signals that manage device operations.

- Registers and Array: The control logic directs information to the internal command register, address register, data register, and cache register, which interact directly with the NAND Flash array (which is divided into multiple planes). The device also includes an internal Status register and ECC (Error Correction Code) circuit, which links to the control logic.

3. How the NAND Flash Communicates

Communication relies on the Open NAND Flash Interface (ONFI) 1.0-compliant protocol and an asynchronous interface bus operation.

A. Multiplexed Bus Operation

The device minimizes pin count by multiplexing data, addresses, and commands onto the shared I/O bus.

- Commands and Addresses: These are always supplied on I/O[7:0].

- Data: Data is input on I/O[7:0] for x8 devices and on I/O[15:0] for x16 devices.

B. Command and Address Transfer Cycles

The device interprets the information on the I/Ox pins based on the state of the control signals, which determines the asynchronous interface mode:

| Mode | CE# | CLE | ALE | WE# | Function |
| --- | --- | --- | --- | --- | --- |
| Command Input | L | H | L | H | Writes an 8-bit command from I/O[7:0] to the command register on the rising edge of WE#. |
| Address Input | L | L | H | H | Writes an 8-bit address from I/O[7:0] to the address register on the rising edge of WE#. |
| Data Input | L | L | L | H | Writes data from I/O[7:0] (or I/O[15:0]) to the cache register on the rising edge of WE#. |

C. Data Output and Status Monitoring

Data output occurs when the device is in a READY state.

- Data Output: Data is output from the cache register to the I/Ox pins on the falling edge of RE# when CE#, ALE, and CLE are LOW, and WE# is HIGH.

- Operation Status: The host monitors the R/B# signal (hardware method) or reads the 8-bit status register (software method, using commands like READ STATUS (70h) or READ STATUS ENHANCED (78h)) to detect operation completion, pass/fail conditions, and write-protect status.

D. Operational Flow

Communication involves defined command sequences (ONFI NAND Flash Protocol). For example:

- Initialization: The first command issued after power-on must be RESET (FFh).

- Program Operation: A typical sequence to program a page includes writing the 80h command, followed by five address cycles (column/row/block), data input, and concluded by the 10h command.

- Advanced Operations: The device supports complex multi-cycle command sets to perform actions like Program page cache mode (80h-15h) or Two-plane commands (e.g., PROGRAM PAGE TWO-PLANE 80h-11h) for improved performance, which require precise timing and sequencing dictated by the controller.

This response is structured based on the major topics and subtopics contained within the Micron NAND Flash Memory data sheet, explaining their purpose, function, and critical role within a Solid State Recorder (SSR) system.

I. General Information, Features, and Architecture

| Topic/Subtopic | Why it Exists & What it Does | Role in SSR System | Citations |
| --- | --- | --- | --- |
| NAND Flash Memory Features | Provides a summary of the core capabilities, such as Single-Level Cell (SLC) technology, Open NAND Flash Interface (ONFI) 1.0-compliance, memory organization (e.g., 2 planes per device), and high-speed asynchronous I/O performance (e.g., $t_{RC}/t_{WC}$ of 20ns for 3.3V). | Establishes the foundation of the SSR's storage medium, defining the maximum capacity, expected performance, and the operational standard (ONFI) the host controller (FPGA/FX3) must support. |  |
| Part Numbering Information | Explains how to decode the Micron part number to determine key physical characteristics, including Density (4Gb, 8Gb, 16Gb), Device Width (x8 or x16), Operating Voltage (1.8V or 3.3V), and Package Code (TSOP or VFBGA). | Necessary for the SSR hardware designer to select and verify the correct memory components compatible with the system's power budget and physical constraints. |  |
| Important Notes and Warnings | Automotive Applications and Critical Applications sections highlight legal disclaimers, asserting that the component vendor is not liable if the product is used in life-critical systems without proper safety design measures implemented by the customer. | Emphasizes the SSR manufacturer's sole responsibility to design robust system safeguards, like redundancy and sophisticated error recovery, especially if the SSR is used in aerospace or critical industrial applications. |  |
| General Description | Explains the basic operational characteristics, specifically detailing the asynchronous data interface and the multiplexed 8-bit bus (I/Ox) used for commands, addresses, and data, minimizing pin count. | Guides the basic electrical design, ensuring the SSR's controller hardware (FPGA/FX3) connects properly via the low pin-count interface. |  |
| Signal Descriptions | Defines the function of the main control pins, such as ALE (Address Latch Enable), CLE (Command Latch Enable), WE# (Write Enable), and R/B# (Ready/Busy output). | Provides the instruction set for the SSR's digital interface logic, indicating precisely which signals must be toggled, and in what sequence, to communicate with the device. |  |
| Signal Assignments and Package Dimensions | Provides the pinout or ball map for different package types (e.g., 48-pin TSOP Type 1, 63-ball VFBGA) and their physical dimensions. | Vital for the physical realization of the SSR's circuit board, ensuring correct component placement and routing connectivity. |  |
| Architecture | Describes the internal data path, including how commands, addresses, and data flow through the command register, address register, and the paired data register and cache register to the NAND Flash array (which is divided into two planes). | Crucial for designing efficient firmware; informs the SSR controller how to leverage the separate data/cache registers during high-speed cache operations to increase throughput. |  |
| Device and Array Organization | Specifies the physical memory layout, including page size (e.g., 2048 main bytes + 64 spare bytes for x8), block size (64 pages), and the structure of the five address cycles used to target a specific block, page, and column address. | Dictates the data management structure for the SSR system; the file system layer must align data transfers with the page size and calculate the correct 5-cycle address sequences for every read or write request. |  |

II. Interface Operations and Initialization

| Topic/Subtopic | Why it Exists & What it Does | Role in SSR System | Citations |
| --- | --- | --- | --- |
| Asynchronous Interface Bus Operation | Explains the mechanism for sharing the I/Ox pins by using control pins (CLE, ALE) to designate whether the bus currently carries a command, an address, or data. | Defines the protocol state machine implemented by the SSR's FPGA/GPIF logic for sequencing low-level transactions. |  |
| Asynchronous Commands/Addresses/Data Input/Output | Details the fundamental actions: input is latched on the rising edge of WE#; data output occurs on the falling edge of RE#. Also includes timing constraints for data latching. | Provides the necessary precise timing data (setup, hold, pulse widths) that the high-speed controller logic must strictly adhere to for reliable data exchange. |  |
| Write Protect# (WP#) | A hardware input signal that, when asserted LOW, disables all PROGRAM and ERASE operations globally. Toggling it LOW during an operation acts like a hardware RESET (FFh) command. | Provides an immediate hardware safety mechanism, ensuring that valuable stored data cannot be accidentally erased during power fluctuations or unexpected system states. |  |
| Ready/Busy# (R/B#) | An open-drain output signal used by the device to signal when it is busy executing an array operation (LOW) or ready to accept a new command (HIGH). This signal is typically tied to an interrupt pin. | Essential for system responsiveness and flow control; the SSR controller monitors this signal to manage hardware interrupts and maximize command efficiency without prematurely issuing commands while the array is active. |  |
| Device Initialization | Specifies the required sequence for power-up, including waiting for R/B# to become valid and HIGH, and then issuing the mandatory RESET (FFh) command as the first operation. | Defines the required boot sequence for the SSR's memory access software to ensure the device starts in a clean, predictable state, preventing data corruption during startup. |  |

III. Commands: Identification and Configuration

| Topic/Subtopic | Why it Exists & What it Does | Role in SSR System | Citations |
| --- | --- | --- | --- |
| Command Definitions | Comprehensive tables listing all available commands (e.g., Program, Erase, Read) and the number of address cycles required for each. | Provides the necessary reference for the SSR firmware developers to correctly construct command sequences for all memory operations. |  |
| Reset Operations (FFh) | Clears the command register and marks internal cache data invalid, aborting any active operation (Read, Program, or Erase). | Enables robust error recovery; if the SSR detects a protocol error or hangs during a memory operation, issuing a RESET (FFh) quickly forces the memory device back to an idle state. |  |
| Identification Operations (90h, ECh, EDh) | Commands used to retrieve crucial metadata: READ ID (90h) returns the manufacturer and device configuration. READ PARAMETER PAGE (ECh) returns the full ONFI parameter page, detailing all timing, capacity, and feature support. READ UNIQUE ID (EDh) retrieves a permanent serial number. | Allows the SSR's boot software to automatically discover the device characteristics, enabling flexible driver implementation and unique device tracking (via the Unique ID). |  |
| Feature Operations (EEh, EFh) | SET FEATURES (EFh) is used to set volatile configurations (e.g., Timing Mode (01h) or Array Operation Mode (90h), which enables/disables internal ECC). GET FEATURES (EEh) reads the current settings. | Essential for performance tuning and reliability configuration. The SSR uses SET FEATURES to enable internal ECC for better error protection or optimize I/O timing based on controller speed. |  |
| Status Operations (70h, 78h) | READ STATUS (70h) reports the status of the last selected die (LUN). READ STATUS ENHANCED (78h) selects a specific die/LUN to report status, necessary to avoid bus contention in multi-die configurations. | Provides the feedback mechanism for the SSR, allowing firmware to poll the FAIL bit to confirm program/erase success and monitor the RDY bit to manage command queuing. |  |

IV. Commands: Data Manipulation

| Topic/Subtopic | Why it Exists & What it Does | Role in SSR System | Citations |
| --- | --- | --- | --- |
| Column Address Operations | Commands like RANDOM DATA READ (05h-E0h) and RANDOM DATA INPUT (85h) allow the host to change the read/write starting column address within a page's cache register. | Enables the SSR controller to access small portions of data (like partial system configuration records or small I/O packets) without having to read or write the full 2112-byte page sequentially. |  |
| Read Operations | Includes standard READ PAGE (00h-30h) and high-throughput Read Cache Operations (31h, 00h-31h, 3Fh), which allow data output from one page while the subsequent page is concurrently fetched from the array. | Defines how recorded data is read back to the host. Cache operations are key to maximizing the SSR's read bandwidth for high-speed playback. |  |
| Program Operations | Includes standard PROGRAM PAGE (80h-10h) and PROGRAM PAGE CACHE (80h-15h), which allows the host to load the next page's data into the cache register while the previous page is still programming in the array. | Essential for writing data to the memory. Cache mode is vital for maintaining sustained high-speed recording rates, as it overlaps I/O latency with internal program time ($t_{PROG}$). |  |
| Erase Operations | Commands like ERASE BLOCK (60h-D0h) wipe a specified block clean, which must be done before a block can be programmed with new data. | Part of the mandatory housekeeping for the SSR, ensuring memory blocks are properly prepared for data overwriting as required by NAND technology. |  |
| Internal Data Move Operations | Uses READ FOR INTERNAL DATA MOVE (00h-35h) and PROGRAM FOR INTERNAL DATA MOVE (85h-10h) to relocate data pages between blocks entirely within the device, without transferring the data over the external I/O bus. | Improves the efficiency of the SSR's wear-leveling and bad block management algorithms by minimizing external bus traffic when relocating data blocks. |  |

V. Protection and Advanced Features

| Topic/Subtopic | Why it Exists & What it Does | Role in SSR System | Citations |
| --- | --- | --- | --- |
| Block Lock Feature | A feature that protects defined ranges of memory blocks from being programmed or erased, using commands like UNLOCK (23h-24h) and LOCK TIGHT (2Ch), the latter of which locks the configuration until the device is power cycled. | Used to physically protect the SSR's boot firmware or critical data tables from accidental modification during field operation or system updates. |  |
| One-Time Programmable (OTP) Operations | Provides a dedicated, protected area (30 pages) for storing data that can be written once and then permanently sealed (OTP DATA PROTECT) to prevent further programming. | Ideal for storing permanent system identifiers, serialization data, or cryptographic keys required by the SSR, as this data is guaranteed immutable. |  |
| Two-Plane Operations | Utilizes the two independent memory planes within a die to perform two READs, PROGRAMs, or ERASEs concurrently, commanded via multi-cycle commands (e.g., READ PAGE TWO-PLANE (00h-00h-30h)). | Maximizes the sustained throughput of the SSR by doubling the number of concurrent array operations, which is essential for high-performance data acquisition. |  |
| Interleaved Die (Multi-LUN) Operations | Allows the host to interleave operations across multiple dies (LUNs) on the same chip enable line, executing an operation on an idle die while another is busy. | Increases the overall system parallelism and capacity utilization, directly translating to higher sustained I/O bandwidth for the SSR. |  |
| Error Management / Internal ECC and Spare Area Mapping | Requires the host to implement bad block management and use Error Correction Code (ECC) algorithms (min required ECC is 4-bit per 528 bytes) to ensure data integrity. It defines how the optional internal ECC engine (which detects 5-bit and corrects 4-bit errors) works. | The foundational layer of reliability for the SSR, forcing the controller design to account for inevitable block failure and to implement ECC to maintain the integrity of recorded data over the device's lifetime. |  |

VI. Electrical and Timing Specifications

| Topic/Subtopic | Why it Exists & What it Does | Role in SSR System | Citations |
| --- | --- | --- | --- |
| Electrical Specifications – DC Characteristics and Operating Conditions | Defines the required power supply voltages (e.g., $V_{CC}$ min/max) and the maximum current draw during various operations (e.g., $I_{CC1}$ READ current, $I_{CC3}$ ERASE current). | Used by SSR power supply designers to ensure stable power delivery and proper thermal management under worst-case operational loads. |  |
| Electrical Specifications – AC Characteristics and Program/Erase Characteristics | Specifies critical timing requirements, such as minimum READ cycle time ($t_{RC}$) of 20ns, minimum WRITE cycle time ($t_{WC}$) of 20ns, and maximum duration of internal processes (e.g., $t_{PROG}$ max 600µs, $t_{BERS}$ max 3ms). | These values are the definitive configuration parameters for the SSR's high-speed interface (FPGA/GPIF), ensuring that the controller hardware operates fast enough but adheres to the memory's timing constraints. |  |
| Asynchronous Interface Timing Diagrams | Provides detailed visual waveforms of the required control signal sequencing and timing relationships for all major operations (e.g., RESET, READ PAGE, PROGRAM PAGE). | Serves as the blueprint for developing and verifying the low-level digital logic (like the FX3 GPIF state machine or FPGA state machine) that controls the memory interface. |  |
| Revision History | Documents all technical and non-technical changes made to the specification over time. | Critical for long-term product support and managing hardware revisions within the SSR program, ensuring that drivers and systems remain compatible across manufacturing batches. |  |