# Digital Design Foundations

This document consolidates and organizes everything we discussed about sequential logic, from basic memory concepts to practical top-level design thinking. It is written to help you think like a digital designer, not just memorize theory.

## 1. Why Sequential Circuits Exist

### Combinational vs Sequential

- Combinational circuit: Output depends only on present inputs.

- Sequential circuit: Output depends on present inputs and past outputs.

Past output = memory → this is what makes a circuit sequential.

Examples of sequential systems: - Counters - Registers - Finite State Machines (FSMs) - Memory interfaces (NAND Flash, FX3, etc.)

## 2. What Is a State?

### Definition

A state is the information that describes where the system is right now.

- Stored inside flip-flops

- Changes only at clock edges

### Present State vs Next State

- Qₙ (present state): what is stored now

- Qₙ₊₁ (next state): what will be stored after the next clock edge

Key rule:

Present State + Inputs → Next State

## 3. Memory Elements

### Latch

- Level-sensitive

- Output follows input while enable is active

- Can change multiple times during enable period

Problem: - Sensitive to glitches - Hard to control in large systems

## 4. Clock – The Time Controller

### What is a Clock?

- A periodic signal: LOW → HIGH → LOW

- Defines when state changes are allowed

Important properties: - Time period (T) - Frequency (f = 1/T) - Duty cycle (usually 50%)

Golden rule: > Clock controls WHEN state changes, not WHAT state becomes.

## 5. Flip-Flops (Edge-Controlled Memory)

Flip-flops solve latch problems by allowing state changes only at clock edges.

### Why Flip-Flops Are Used

- No accidental changes

- Predictable timing

- Safe for large designs

## 6. SR Flip-Flop (Conceptual Learning Tool)

Inputs: S (Set), R (Reset)

| S | R | Q(next) |
| --- | --- | --- |
| 0 | 0 | Memory |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Invalid |

Problem: - Invalid state when S=R=1

SR flip-flop is important to understand memory, but not ideal for real designs.

## 7. D Flip-Flop – The Most Important Flip-Flop

### Key Idea

Qₙ₊₁ = D (at clock edge)

Inputs: - D (data) - Clock

Behavior: - Clock LOW → memory - Clock edge → output copies D

### Why D Flip-Flop Is King

- No invalid states

- Single input

- Easy to use in FSMs

- Used internally by FPGA hardware

### Mental Model

“Whatever is on D at the clock edge gets written into memory.”

## 8. Characteristic & Excitation Tables (Meaning, Not Memorization)

### Characteristic Table

- Tells next state given present state and inputs

For D flip-flop:

Qₙ₊₁ = D

### Excitation Table

- Tells what input is needed to move from Qₙ to Qₙ₊₁

For D flip-flop:

D = Qₙ₊₁

These tables are design tools for FSMs and counters.

## 9. JK Flip-Flop – Why It Exists

JK flip-flop improves SR by fixing the invalid state.

| J | K | Qₙ₊₁ |
| --- | --- | --- |
| 0 | 0 | Qₙ |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Q̅ₙ (Toggle) |

Advantage: - Toggle operation built-in

Problem: - When clock is level-high and J=K=1 → race-around condition

## 10. Race-Around vs Toggling

### Race-Around

- Multiple output changes within one clock pulse

- Uncontrolled

- Caused by feedback + level-sensitive clock

### Toggling

- Exactly one output change per clock cycle

- Controlled

- Desired behavior

## 11. Master–Slave Flip-Flop

### Core Idea

Use two flip-flops so feedback cannot act continuously.

Structure: - Master FF → driven by CLK - Slave FF → driven by inverted CLK

Key properties: - Master active when CLK=1 - Slave active when CLK=0 - Never active at the same time

Result: - Race-around eliminated - Output changes once per cycle

## 12. Master–Slave ≡ Negative Edge Triggered

Because: - Slave updates output only when CLK goes HIGH → LOW

Conclusion: > Master–Slave flip-flop behaves like a negative-edge-triggered flip-flop

## 13. Glitches and How They Are Removed

### Why Glitches Occur

- Input changes while clock is active

- Level-sensitive stages pass changes immediately

### Why Slave Output Is Clean

- Slave samples master only when clock is LOW

- Master output is stable at that time

Thus: > Master–Slave architecture filters glitches

## 14. What to Remember for Top-Level Design

### The Golden Rules

- Think in states, not gates

- State changes only at clock edges

- Use D flip-flops for implementation

- Separate logic clearly:

  - State register

  - Next-state logic

  - Output logic

- Avoid gated clocks

- Respect setup and hold times

- Inputs must not change during active clock window

- Behavioral simulation checks logic, not physical timing

## 15. How This Applies to Real Systems (NAND Flash Example)

Control signals like: - CLE, ALE, WE#, CE#, RE#

Internally do: - Select which register (D FF bank) is enabled - Use WE# edge as a clock - Latch I/O data into internal registers

Conceptually:

if WE# rising edge and enable conditions:
    register <= I/O

That is pure D flip-flop + clock + enable logic.

## Final Takeaway

Gates build flip-flops. Flip-flops store states. States build systems.

If you think in states and clock edges, everything else becomes simple.